SMALL VOLUME MANUFACTURER

JOINT COMMENT ON 15 DAY NOTICE

ADVANCED CLEAN CAR CARB PROPOSAL

March 5, 2012

This comment is submitted on behalf of Aston Martin Lagonda Limited, Lotus Cars Limited and McLaren Automotive Limited. Each of these companies is classified as a “small volume manufacturer” (SVM)  under CARB regulations and is a manufacturer of a very limited number of high performance sports cars. See www.astonmartin.com; www.lotuscars.com; www.mclarenautomotive.com

We support the proposed changes in CARB’s 15 day notice dated February 22, 2012.  The proposed modified text adds clarity to the timing of the FTP and SFTP requirements for SVMs.

Set forth below is our current understanding of the LEVIII SVM requirements.   Portion in yellow indicates area changed per CARB 15 day notice

- EXHAUST LEVIII STANDARDS

- FTP FLEET SVM STANDARDS   NMOG+NOx (g/mi)

| Model Year | NOx+NMOG | DURABILITY (FULL USEFUL LIFE) | CERTIFICATION FUEL |
| --- | --- | --- | --- |
| Through 2021 | 0.160 (current req.) | 120k miles | Existing |
| 2022-24 | 0.125 -ULEV 125 | 150k | E10 |
| 2025 and + | 0.070 (ULEV 70) | 150k | E10 |

- Particulate Standards for Small Volume Manufacturers.

    - 2021 - 2027 model years -- certify 100 percent of its passenger car vehicle fleet to the 3 mg/mi particulate standard. 2028 and subsequent model years -- certify 100 percent of its passenger car vehicle fleet to the 1 mg/mi particulate standard

- CARB agrees to revise and keep updated assigned DFs

- SFTP II  STANDARDS

| Model Year | SVM Requirements |  | DURABILITY | CERTIFICATION FUEL |
| --- | --- | --- | --- | --- |
| Through 2021 | SFTP I LEVII existing level |  | 4k miles | Existing |
| 2022+ | SFTP II ULEV/ULEV70/ULEV50 level |  | 150k miles | E10 |
|  | US06 | SC03 |  |  |
|  | NMHC+NOx 0.120 g/mi CO 9.6 g/mi PM 10 mg/mi | NMHC+NOx 0.070 g/mi CO 3.2 g/mi |  |  |

- CARB agrees to revise and keep updated assigned DFs

- Composite standards may also be adopted but typically with few test groups, SVMs would tend not to use such

- EVAPORATIVE LEVIII  STANDARDS

| Model Year | SVM REQUIREMENT |  | DURABILITY | CERTIFICATION FUEL |
| --- | --- | --- | --- | --- |
| Through 2021 | LEVII standard |  | 120k | existing |
| 2022 and + | LEVIII standard |  | 150k | E10 |
| Option 1 |  |  |  |  |
| Running Loss g/mi | 3 day and 2 day Diurnal / Hot Soak |  |  |  |
|  | Whole vehicle g/test | Fuel Only g/test – see note below |  |  |
| 0.05 | 0.350 | 0.0 |  |  |
| Option 2 |  |  |  |  |
| Running Loss g/mi | Highest Whole Vehicle Diurnal + Hot Soak g/test | Canister Bleed Test g/test |  |  |
| 0.05 | 0.300 | 0.020 |  |  |

Note -- In lieu of demonstrating compliance with the fuel-only emission standard (0.0 grams per test) over the three-day and two-day diurnal plus hot soak tests, a manufacturer may, with advance Executive Officer approval, demonstrate compliance through an alternate test plan.

- CARB agrees to revise and keep updated assigned DFs

- Procedure for deduction of background evaporative emissions from full vehicle testing (mirror image of rig testing) continue to be accepted; an important issue to SVMs given SVM usage of composites, leather and larger tires

Thank you.

Sincerely,

Lance Tunick

P.O. Box 23078 - Santa Fe - NM 87502-3078 - USA

Tel.  (505) 570 1845

tunick @vsci.net