# Appendix A-2

Proposed Regulation Order

Proposed Amendments to the California Reformulated Gasoline Regulations

[Note: This version of the Proposed Regulation Order is provided in a tracked changes format to improve the accessibility of the regulatory text. This version is not the authoritative version for this proposed rulemaking. The proposed amendments are incorporated into the current regulatory text for ease of readability only. For the authoritative version that complies with Government Code section 11346.2, subdivision (a)(3), please see Appendix A-1. To review this document in a clean format (no underline or strikeout to show changes), please accept all tracked changes. Subsections for which no changes are proposed in this rulemaking are indicated with
 “*    *    *    *.”]

Chapter 5. Standards for Motor Vehicle Fuels

Article 1. Standards for Gasoline

Subarticle 1. Gasoline Standards that Became Applicable Before 1996

Section 2250. 	Degree of Unsaturation for Gasolines Sold Before April 1, 1996.

Section 2251.5. 	Reid Vapor Pressure of Gasoline Sold After January 1, 1992, and Before March 1, 1996.

Section 2252. 	Sulfur Content of Gasoline Represented as Unleaded Sold Before April 1, 1996.

Section 2253.4. 	Lead in Gasoline.

Section 2257. 	Required Additives in Gasoline.

Subarticle 2. Standards for Gasoline Sold Beginning March 1, 1996

Section 2260.	Definitions

Section 2261. 	Applicability of Standards; Additional Standards.

Section 2262. 	The California Reformulated Gasoline Phase 2 and Phase 3 Standards.

Section 2262.3.	Compliance with CaRFG Phase 2 and CaRFG Phase 3 Standards for Sulfur, Benzene, Aromatic Hydrocarbons, Olefins, T50 and T90.

Section 2262.4.	Compliance with CaRFG Phase 2 and CaRFG Phase 3 Standards for Reid Vapor Pressure.

Section 2262.5. 	Compliance with the Standards for Oxygen Content.

Section 2262.6. 	Prohibition of MTBE and Oxygenates Other than Ethanol in California Gasoline Starting December 31, 2003.

Section 2262.9.	Requirements Regarding Denatured Ethanol Intended For Use as a Blend Component in California Gasoline.

Section 2264.	Designated Alternative Limits.

Section 2265. 	Gasoline Subject to PM Alternative Specifications Based on the California Predictive Model.

Section 2265.1.	Offsetting Emissions Associated with Higher Sulfur Levels.

Section 2265.5. 	 Alternative Emission Reduction Plan (AERP).

Section 2266. 	Certified Gasoline Formulations Resulting in Equivalent Emission Reductions Based on Motor Vehicle Emissions Testing.

Section 2266.5. 	Requirements Pertaining to California Reformulated Blendstock for Oxygen Blending (CARBOB) and Downstream Blending.

Section 2269.	Submittal of Compliance Plans.

Section 2270. 	Testing and Recordkeeping.

Section 2271. 	Variances.

Section 2272. 	CaRFG Phase 3 Standards for Qualifying Small Refiners.

Section 2273. 	Labeling of Equipment Dispensing Gasoline Containing MTBE.

Article 2. Standards for Diesel Fuel

Section 2282. 	Aromatic Hydrocarbon Content of Diesel Fuel.

Proposed Regulation Order

Title 13, California Code of Regulations

Amend Sections 2253.4, 2257, 2260, 2261, 2262, 2262.3, 2262.4, 2262.5, 2262.6,  2262.9, 2264, 2265, 2265.1, 2266.5, 2270, 2271, 2272, and 2282, and Repeal Sections 2250, 2251.5, 2252, 2265.5, 2266, 2269, and 2273 of title 13, California Code of Regulations, to read as follows:

SUBARTICLE 1. GASOLINE STANDARDS THAT BECAME APPLICABLE BEFORE 1996

-

# 2253.4. Lead in Gasoline.

## Regulatory Standard.

### Starting January 1, 1994, no person shall sell, offer for sale, supply, or offer for supply any California gasoline:

#### which is not represented as unleaded, or

#### which has been produced with the use of any lead additive, or

#### which contains more than 0.050 gram of lead per gallon or more than 0.005 gram of phosphorus per gallon, except as provided in subsection (d).

### Starting January 1, 1992, no person shall transfer a consumer gasoline additive containing lead into the fuel tank of a motor vehicle, other than an exempt off-road motor vehicle.

### Starting January 1, 1992, no person shall sell or offer for sale a consumer gasoline additive containing lead unless the additive container bears a conspicuous legend that use of the additive in passenger cars and other on-road vehicles is unlawful and can result in substantial penalties, and unless the marketing of the additive is directed exclusively towards use in exempt off-road motor vehicles and nonvehicular sources.

## Definitions.

For the purposes of this section:

### “California gasoline” means gasoline sold or intended for sale as a motor vehicle fuel in California.

### “Consumer gasoline additive” means any gasoline additive which is designed or marketed to be dispensed into the gasoline tank used to fuel a gasoline engine.

### “Exempt off-road vehicle” means any special construction equipment as defined in sections 565 and 570 of the Vehicle Code, and any implement of husbandry as defined in sections 36000 et seq. of the Vehicle Code.

### “Gasoline” means any fuel which is commonly or commercially known or sold as gasoline, or which is a mixture of any fuel commonly known or sold as gasoline and alcohol.

### “Lead additive” means any substance containing lead or lead compounds.

### “Motor vehicle” has the same meaning as defined in section 415 of the Vehicle Code.

### “Retail outlet” means any establishment at which gasoline is sold or offered for sale to the general public for use in motor vehicles other than exempt off-road vehicles.

### “Supply” means to provide or transfer a product to a physically separate facility, vehicle, or transportation system.

## Test Methods.

The lead content of gasoline shall be determined in accordance with ASTM D3237-79, which is incorporated herein by reference. The phosphorous content of gasoline shall be determined in accordance with ASTM Method D3231-73, which is incorporated herein by reference.

## Exemptions.

Subsection (a)(1) shall not apply to California gasoline sold, offered from sale, supplied, or offered for supply by a person who demonstrates that:

### The gasoline is conspicuously identified as a fuel which may not lawfully be dispensed to motor vehicles other than exempt off-road vehicles; and

### He or she has taken reasonable precautions to assure that the gasoline will not be sold or offered for sale at a retail outlet; and

### Either the gasoline is being directly dispensed into the fuel tank of an exempt off-road vehicle, or the gasoline is the subject of a declaration under penalty of perjury by the purchaser, offeree or recipient stating that he or she will not sell, offer for sale, supply, or offer for supply the gasoline for use in motor vehicles other than exempt off-road vehicles.

Note: Authority cited: Sections 39600, 39601, 43013, 43018 and 43101, Health and Safety Code. Reference: Sections 39000, 39001, 39002, 39003, 39500, 43000, 43013, 43016, 43018, and 43101, Health and Safety Code; and Western Oil and Gas Ass'n. v. Orange County Air Pollution Control District, 14 Cal.3d 411, 121 Cal. Rptr. 249 (1975).

*    *    *    *

# 2257. Required Additives in Gasoline.

## Regulatory Standard.

### On or after January 1, 1992, no person shall sell, offer for sale, supply, or offer for supply any California gasoline unless at the time of the transaction:

#### the producer, importer, or distributor of the gasoline has been issued a currently effective certification for California gasoline pursuant to subsection (c), originally dated no earlier than July 1, 1996, and

#### the gasoline contains at least the minimum concentration of the additive or additives identified in the final application for certification.

### Subsection (a)(1) shall not apply to transactions where the person selling, supplying, or offering the gasoline demonstrates that:

#### the gasoline has not yet been sold, offered, or supplied from the final distribution facility, and either

#### the person has taken reasonably prudent precautions to assure that he or she will bring the gasoline into satisfaction with the requirements of subsection (a)(1) before it is sold, supplied or offered from the final distribution facility, or

#### at or before the time of the transaction the person has obtained a written statement from the purchaser, recipient, or offeree of the gasoline stating that he or she is a distributor who has been issued a currently effective certification pursuant to subsection (c), and will cause the gasoline to satisfy the requirements of subsection (a)(1) before it is sold, supplied or offered from the final distribution facility.

### Subsection (a)(1)[ii] shall not apply to the sale, supply, or offer of gasoline from a final distribution facility where the person selling, supplying, or offering the gasoline demonstrates that the gasoline will be corrected to comply with section (a)(1)[ii] prior to the sale of gasoline from the retail outlet to be dispensed into motor vehicles. If such corrective action is taken, the producer, importer, or distributor of the gasoline must notify the executive officer by telephone or in writing within 2 business days of the correction and must maintain records to document each occurrence in accordance with subsection (d).

### For the purposes of subsection (a)(1), each sale of gasoline at retail for use in a motor vehicle, and each supply of gasoline into a motor vehicle fuel tank, shall also be deemed a sale or supply by any person who previously sold or supplied such gasoline in violation of subsection (a)(1).

## Definitions.

For the purposes of this section:

### “Additive” means any substance or mixture of substances that is intentionally added to gasoline for the purpose of reducing or preventing fuel injection system or intake valve deposits, and that is not intentionally removed prior to the gasoline's sale or use.

### “Bulk purchaser-consumer” means a person who purchases or otherwise obtains gasoline in bulk and then dispenses it into the fuel tanks of motor vehicles owned or operated by the person.

### “California gasoline” means gasoline sold or intended for sale as a motor vehicle fuel in California.

### “Chemical composition” means the name, percentage by weight, and chemical identification of each compound in an additive.

### “Distributor” means any person who transports or stores or causes the transportation or storage of gasoline, produced or imported by another person, at any point between any producer's or importer's facility and any retail outlet or wholesale purchaser-consumer's facility.

### “Final distribution facility” means the stationary gasoline transfer point from which gasoline is transferred into the cargo tank truck, pipeline, or other delivery vessel from which the gasoline will be delivered to the facility at which the gasoline will be dispensed into motor vehicles.

### “Gasoline” means any fuel which is sold or intended for sale as a California motor vehicle fuel and is either: (a) commonly or commercially known or sold as gasoline, or (b) any fuel blend of gasoline as defined in (a) and alcohol in which the portion of gasoline is more than 50 percent of the total blend.

### “Gasoline production facility” means a facility in California at which gasoline is produced; it does not include a facility whose sole operation is to transfer gasoline or to blend additives into gasoline.

### “Importer” means any person who first accepts delivery of gasoline in California.

### “Import facility” means the facility at which imported gasoline is first received in California, including, in the case of gasoline imported by cargo tank and delivered directly to a facility for dispensing gasoline into motor vehicles, the cargo tank in which the gasoline is imported.

### “Motor vehicle” has the same meaning as defined in section 415 of the Vehicle Code.

### “Produce” means to convert liquid compounds which are not gasoline into gasoline.

### “Producer” means any person who produces California gasoline in California.

### “Retail outlet” means any establishment at which gasoline is sold or offered for sale for use in motor vehicles.

### “Supply” means to provide or transfer a product to a physically separate facility, vehicle, or transportation system.

## Certification Requirements.

#### No gasoline formulation shall be certified under this subsection (c) unless the applicant for certification demonstrates each of the following to the executive officer's satisfaction:

##### The gasoline formulation meets a maximum of 50 milligrams averaged over all intake valves when tested in accordance with ASTM D 5500-98, which is incorporated herein by reference. As an alternative, intake valve deposits may be tested in accordance with subsection (c)(1)(A)(iii).

##### The gasoline formulation does not result in a flow loss of more than five percent for any fuel injector when tested in accordance with ASTM D 5598-95a, which is incorporated herein by reference.

##### The gasoline formulation meeting the requirements of (c)(1)(A)(i), does not result in more than 1300 milligrams total deposit weight, averaged over all four combustion chambers, or, does not result in more than 140 percent total deposit weight from all four combustion chambers, relative to the gasoline formulation containing no additive, when tested in accordance with CARB's Test Method for Evaluating Intake Valve and Combustion Chamber Deposits in Vehicle Engines, dated March 12, 1999, which is incorporated herein by reference.

*    *    *    *

Note: Authority cited: Sections 39600, 39601, 43013, 43018 and 43101 of the Health and Safety Code. Reference: Sections 39000, 39001, 39002, 39003, 39500, 39515, 39516, 41511, 43000, 43016, 43018 and 43101, Health and Safety Code; and Western Oil and Gas Ass'n. v. Orange County Air Pollution Control District, 14 Cal.3d 411, 121 Cal. Rptr. 249 (1975).

*    *    *    *

*    *    *    *

# 2259. Exemptions for Motor Vehicle Fuels Used in Test Programs.

### Any person may request an exemption for fuel used in a test program by submitting an application to the executive officer that includes all the information listed in sections (c), (d), (e), and (f).

### For the purpose of this section, “fuel requirement” means any requirement for a motor vehicle fuel established in Chapter 5 (Standards for Motor Vehicle Fuels) of Division 3, Title 13, California Code of Regulations.

### For the purpose of this section, “exemption” means an exemption from one or more fuel requirements that is granted by the executive officer for the purpose of research, motor vehicle or engine emissions certification, fuel certification or registration, or fuel additive certification or registration.

### For the purpose of this section, “test track” means a roadway that is closed to the general public, is used to test motor vehicles or motor vehicle fuels, and is not used to transport persons or property.

### In order for an exemption to be granted, the applicant must demonstrate the following:

*    *    *    *

### Sections (c), (d), (e), and (f) describe what constitutes a sufficient demonstration for each of the four elements in sections (b)(1)(A) through (D).

### Within 20 days of receipt of an application for an exemption, the executive officer shall advise the applicant in writing either that the application is complete or that specified additional information is required to make it complete. Within 15 days of submittal of additional information, the executive officer shall advise the applicant in writing either that the information submitted makes the application complete or that specified additional information is still required to make it complete. Within 20 days after the application is deemed complete, the executive officer shall grant or deny the application. Any denial shall be accompanied by a written statement of the reasons for denial.

*    *    *    *

## With respect to control, a program must be capable of affording the executive officer a monitoring capability. At a minimum, the application for exemption must also include:

### The technical nature of the test program;

### The site(s) of the program (including the street address, city, county, and zip code);

### The manner in which information on vehicles and engines used in the program will be recorded and made available to the executive officer;

### The manner in which results of the program will be recorded and made available to the executive officer;

### The manner in which information on the fuel used in the test program (including noncomplying properties, name, address, telephone number, and contact person of supplier, quantity, date received from the supplier) will be recorded and made available to the executive officer;

### The manner in which the distribution pumps will be labeled to insure proper use of the test fuel;

### The name, address, telephone number and title of the person(s) in the organization requesting an exemption from whom further information on the request may be obtained; and

### The name, address, telephone number and title of the person(s) in the organization requesting an exemption who will be responsible for recording and making the information specified in sections (f)(3), (4), and (5)  available to the executive officer and the location in which such information will be maintained.

## An exemption shall be granted by the executive officer upon a demonstration that the requirements of sections (b), (c), (d), (e) and (f) have been met. The exemption will be granted in the form of memorandum of exemption signed by the applicant and the executive officer (or his delegate), which shall include such terms and conditions as the executive officer determines necessary to monitor the exemption and to carry out the purpose of this section. Any violation of such term or condition shall cause the exemption to be void.

## No fuel requirement shall apply to fuel used for an engine or vehicle dynamometer test, or to fuel used in the testing of motor vehicles or motor vehicle fuels on a test track.

Note: Authority cited: Sections 39600, 39601, 43013, 43018 and 43101, Health and Safety Code. Reference: Sections 39000, 39001, 39002, 39003, 39500, 39515, 39516, 41511, 43000, 43016, 43018 and 43101, Health and Safety Code; and Western Oil and Gas Ass'n. v. Orange County Air Pollution Control District, 14 Cal.3d 411, 121 Cal. Rptr. 249 (1975).

SUBARTICLE 2. STANDARDS FOR GASOLINE SOLD

BEGINNING MARCH 1, 1996

# 2260. Definitions.

## For the purposes of this subarticle, the following definitions apply:

### “Air basin” has the same meaning as defined in section 39012 of the Health and Safety Code.

### “Averaging compliance option” means, with respect to a specific gasoline property, the compliance option set forth in section 2262.3(c).

### “ASTM” means ASTM International.

### “Bulk purchaser-consumer” means a person that purchases or otherwise obtains gasoline in bulk and then dispenses it into the fuel tanks or motor vehicles owned or operated by the person.

### “Bulk plant” means an intermediate gasoline distribution facility where delivery of gasoline to and from the facility is solely by truck.

### “California gasoline” means:

#### Gasoline sold, intended for sale, or made available for sale as a motor vehicle fuel in California; and

#### Gasoline that is produced in California, and that the producer knows or reasonably should know will be offered for sale or supply at an out-of-state terminal or bulk plant at which it will be identified as gasoline produced in California and suitable for sale as a motor vehicle fuel in California.

“California reformulated gasoline blendstock for oxygenate blending, or ‘CARBOB,’” means a petroleum-derived liquid which is intended to be, or is represented as, a product that will constitute California gasoline upon the addition of a specified type and percentage (or range of percentages) of oxygenate to the product after the product has been supplied from the production or import facility at which it was produced or imported.

“Cap limit” means the absolute maximum or minimum value allowed for any regulated chemical or physical property of CARBOB and final blends of California gasoline when using the Predictive Model.

### “CaRFG Phase 3” means California Phase 3 reformulated gasoline.

### “CARBOB limits” means, for a final blend of CARBOB, CARBOB specifications for maximum Reid vapor pressure, sulfur content, benzene content, olefin content, aromatic hydrocarbon content, T50 and T90, expressed to the number of significant figures identified for each property in the section 2266.5 standards table.

### “Common carrier pipeline” means a pipeline operating under Public Utilities Commission tariffs which offers refined petroleum product transportation services to any qualified shipper.

### “Designated alternative limit” means an alternative gasoline specification limit, expressed in the nearest part per million by weight for sulfur content, nearest hundredth percent by volume for benzene content, nearest tenth percent by volume for aromatic hydrocarbon content, nearest tenth percent for olefin content, and nearest degree Fahrenheit for T90 and T50, which is assigned by a producer or importer to a final blend of California gasoline pursuant to section 2264.

### “Designated emissions offsetting limit” means an alternative gasoline specification limit, expressed in the nearest hundredth pound per square inch for RVP, nearest tenth percent by weight for oxygen, nearest part per million by weight for sulfur content, nearest hundredth percent by volume for benzene content, nearest tenth percent by volume for aromatic hydrocarbon content, nearest tenth percent for olefin content, and nearest degree Fahrenheit for T90 and T50, which is assigned by a producer or importer that produces gasoline to a final blend of California gasoline pursuant to section 2265.1.

### “Drag reducing agent” means a long chain polymer chemical that is used in crude oil, refined products or non-potable water pipelines injected by the pipeline operator in small amounts (parts per million) and is used to reduce the frictional pressure drop along the pipeline's length.

“E10” means CaRFG Phase 3 gasoline with up to 10.0 percent ethanol by volume that meets all CaRFG Phase 3 gasoline specifications with the oxygen percent by weight limit applicable for CaRFG Phase 3 gasoline with up to 10.0 percent ethanol by volume ethanol.

“E15” means CaRFG Phase 3 gasoline containing 10.1 to 15 percent ethanol by volume that meets all CaRFG Phase 3 gasoline specifications with the oxygen percent by weight limit applicable for CaRFG Phase 3 gasoline containing 10.1 to 15 percent ethanol by volume.

“E85” means fuel ethanol meeting the specifications in California Code of Regulations, title 13, section 2292.4.

### “Ethanol” means ethyl alcohol which meets any additional requirements for ethanol or ethyl alcohol in Health and Safety Code section 43830.

### “Emissions associated with permeation” means the incremental increase in emissions because of permeation which is calculated as the difference between the emissions from the producer's or importer's final blend formulation and the flat limits without ethanol. The Phase 3 reformulated gasoline Predictive Model, as described in the “California Procedures for Evaluating Alternative Specifications for Phase 3 Reformulated Gasoline Using the California Predictive Model” as incorporated by reference in section 2265(a)(2)(A), shall be used to calculate emissions associated with permeation.

Emissions are calculated as follows:

Ozone Forming Potential (tons per day) = 18.4 (tons per day) * (PCE(OFP)/2.39) * 2.80, and

NOx (tons per day) = 427.8 (tons per day) * PCE(NOx) where PCE(OFP) and PCE(NOx) = Percent change in emissions, as predicted by the CaRFG3 Predictive Model for Ozone Forming Potential (OFP) and Oxides of Nitrogen (NOx), respectively, as described in the applicable version of the “California Procedures for Evaluating Alternative Specifications for Phase 3 Reformulated Gasoline Using the California Predictive Model” as incorporated by reference in section 2265(a)(2)(A).

### “Executive Officer” means the executive officer of the California Air Resources Board (CARB), or his or her designee.

### “Final blend” means a distinct quantity of gasoline or CARBOB which is introduced into commerce in California without further alteration which would tend to affect a regulated gasoline specification of the fuel.

### “Final blend credit” means the credit from a final blend of gasoline that may be used to offset a producer's or importer's final blend deficit. The amount of final blend credit shall be calculated as follows:

Final Blend Credit = (PCE - PCEt) * Vc, where

PCE = Percent change in emissions values as reported by the producer or importer.

PCEt = Percent change in emissions values, as they pertain to the Predictive Model emissions offsetting compliance option, for the Predictive Model alternative specifications that the producer or importer was intending to produce and which would have met the criteria for approval in the applicable Predictive Model Procedures but for the elevated sulfur content.

Vc = volume, in barrels, of the final blend that has a final blend credit.

### “Final blend deficit” means the deficit from a final blend of gasoline that a producer or importer must offset. The amount of final blend deficit shall be calculated as follows:

Final Blend Deficit = (PCE - 0.04) * Vd, where

PCE = Percent change in emissions values, as they pertain to the Predictive Model emissions offsetting compliance option, which are greater than 0.04%. If the percent change in emissions values, as they pertain to the Predictive Model emissions offsetting compliance option, are all less than 0.04%, there is no final blend deficit.

Vd = volume, in barrels, of the final blend that has a final blend deficit.

### “Final distribution facility” means the stationary gasoline transfer point from which gasoline or CARBOB is transferred into the cargo tank truck, pipeline, or other delivery vessel from which the gasoline will be delivered to the facility at which the gasoline will be dispensed into motor vehicles; except that a cargo tank truck is the final distribution facility where the cargo tank truck is used to transport CARBOB and gasoline and carries written documentation demonstrating that the designated type and amount or range of amounts of oxygenates designated by the producer or importer will be or have been blended directly into the cargo tank truck prior to delivery of the resulting gasoline from the cargo tank truck to the facility at which the gasoline will be dispensed into motor vehicles.

“Flat limit” means the absolute maximum or minimum value allowed for any regulated chemical or physical property of California gasoline leaving a refinery or import facility without using the Predictive Model.

### “Flat limit compliance option” means, with respect to a specific gasoline property, the compliance option set forth in section 2262.3(b), section 2262.4(b)(1), or section 2262.5(c).

### “Further process” means to perform any activity on gasoline, including distillation, treating with hydrogen, or blending, for the purpose of bringing the gasoline into compliance with the standards in this subarticle.

### “Gasoline” means any fuel that is commonly or commercially known, sold or represented as gasoline, including any volatile mixture of predominantly liquid hydrocarbons that is sold or represented as suitable for use in an automotive spark-ignition engine.

### “Imported gasoline” means gasoline which is transported into California and does not meet the definition for California gasoline that is produced in California, and that the producer knows or reasonably should know will be offered for sale or supply at an out-of-state terminal or bulk plant at which it will be identified as gasoline produced in California and suitable for sale as motor vehicle fuel in California.

### “Import facility” means the storage tank to which imported California gasoline or CARBOB is first delivered in California, including, in the case of gasoline or CARBOB imported by cargo tank and delivered directly to a facility for dispensing gasoline into motor vehicles, the cargo tank in which the gasoline or CARBOB is imported.

### “Importer” means any person who first accepts delivery in California of imported California gasoline.

### “Motor vehicle” has the same meaning as defined in section 415 of the Vehicle Code.

### “Oxygenate” is any oxygen-containing, ashless, organic compound, such as an alcohol or ether, which, when added to gasoline increases the amount of oxygen in gasoline.

### “Oxygenate blending facility” means any facility (including a truck) at which oxygenate is added to gasoline or blendstock, and at which the quality or quantity of gasoline is not altered in any other manner except for the addition of deposit control additives or other similar additives.

### “Oxygenate blender” means any person who owns, leases, operates, controls, or supervises an oxygenate blending facility, or who owns or controls the blendstock or gasoline used or the gasoline produced at an oxygenate blending facility.

### “Percent change in emissions values, as they pertain to the Predictive Model emissions offsetting compliance option” means values calculated, each for oxides of nitrogen, total ozone forming potential, and potency-weighted toxics, from the Phase 3 Predictive Model using the designated emissions offsetting limits for the candidate fuel and the flat limits in section 2262 for the reference fuel, as described in the “California Procedures for Evaluating Alternative Specifications for Phase 3 Reformulated Gasoline Using the California Predictive Model,” the applicable version as described in section 2265(a)(2)(A), which is incorporated by reference in sections 2265(a)(2)(A)(2)-(7).

### “Pipeline tender” means a specific volume of product having a unique name or designation which is offered to a pipeline for transportation.

### “Predictive Model alternative gasoline formulation” means a final blend of gasoline that is subject to a set of Predictive Model alternative specifications assigned pursuant to section 2265(a).

### “Predictive Model alternative specifications” means the specifications for the following gasoline properties, as determined in accordance with section 2263 and expressed to the number of significant figures identified for each property in the section 2262 standards table: maximum Reid vapor pressure, maximum sulfur content, maximum benzene content, maximum olefin content, minimum and maximum oxygen content, maximum T50, maximum T90, and maximum aromatic hydrocarbon content.

### “Predictive Model averaging compliance option” means, with reference to a specific gasoline property, the compliance option for Predictive Model alternative gasoline formulations under which final blends of gasoline are assigned designated alternative limits in accordance with section 2264.

### “Predictive Model averaging limit” means a Predictive Model alternative specification that is subject to the Predictive Model averaging compliance option.

### “Predictive Model emissions offsetting compliance option” means, with respect to a specific gasoline property, the compliance option set forth in section 2265.1(a).

### “Predictive Model emissions offsetting formulation” means a final blend of gasoline that is subject to a set of designated emissions offsetting limits assigned pursuant to section 2265.1(a).

### “Predictive Model flat limit” means a Predictive Model alternative specification that is subject to the Predictive Model flat limit compliance option.

### “Predictive Model flat limit compliance option” means, with reference to a specific gasoline property, the compliance option under which each gallon of gasoline must meet the specification for the property contained in the Predictive Model alternative specifications.

#### “Produce” means, except as otherwise provided below, to convert liquid compounds which are not gasoline into gasoline or CARBOB. When a person blends volumes of blendstocks which are not gasoline with volumes of gasoline acquired from another person, and the resulting blend is gasoline, the person conducting such blending has produced only the portion of the blend which was not previously gasoline. When a person blends gasoline with other volumes of gasoline, without the addition of blendstocks which are not gasoline, the person does not produce gasoline.

#### Where a person supplies gasoline to a refiner who agrees in writing to further process the gasoline at the refiner's refinery and to be treated as the producer of the gasoline, the refiner shall be deemed for all purposes under this article to be the producer of the gasoline.

#### Where an oxygenate blender blends oxygenates into CARBOB which has already been supplied from a gasoline production facility or import facility, and does not alter the quality or quantity of the CARBOB or the resulting gasoline in any other manner except for the addition of deposit control additives or other similar additives, the oxygenate blender is not producing any portion of the resulting gasoline, and the producer or importer of the CARBOB is treated as the producer or importer of the full volume of the resulting gasoline.

### “Produced at a production facility with the use of any oxygenate other than ethanol or MTBE” means produced at a production facility in part by either (i) adding at the production facility any oxygenate, other than ethanol or MTBE, in neat form to the California gasoline or to a blending component used in the gasoline; or (ii) using a blending component that contained greater than 0.10 weight percent total oxygen from oxygenates other than ethanol or MTBE when it was supplied to the production facility.

### “Producer” means any person who owns, leases, operates, controls or supervises a production facility.

### “Production facility” means a facility in California at which gasoline or CARBOB is produced. Upon request of a producer, the executive officer may designate, as part of the producer's production facility, a physically separate bulk storage facility which (A) is owned or leased by the producer, and (B) is operated by or at the direction of the producer, and (C) is not used to store or distribute gasoline or CARBOB that is not supplied from the production facility.

### “Qualifying volume” means, for each small refiner, a volume of gasoline determined in accordance with the following four steps, provided that the qualifying volume for Kern Energy's Bakersfield refinery shall not exceed 2,920,000 barrels per year (equal to 8000 barrels per day; 2,928,000 barrels per year in leap years):

#### First, the barrel per calendar day “operating crude oil capacity” of the small refiner's refinery in March 1999 is identified, based on data which are reported to the executive officer from the California Energy Commission (CEC) and are derived from “Monthly Refinery Reports” (EIA 810) submitted to the CEC no later than June 30, 1999. If the CEC is unable to derive such data from the Monthly Refinery Reports for a particular small refiner, the executive officer shall determine the small refiner's operating crude oil capacity in March 1999 based on other publicly available and generally recognized sources.

#### Second, this operating crude oil capacity is multiplied by 0.9794, representing the highest monthly refinery operating utilization rate in the California refining industry for January 1998 through March 1999, as compiled in the “Monthly Refinery Capacity Data Statewide” report of the CEC.

#### Third, the resulting crude throughput volume is multiplied by the refinery's highest monthly ratio of gasoline produced to crude oil distilled in January 1998 through March 1999, based on data derived by the CEC from the Monthly Refinery Reports submitted to the CEC no later than June 30, 1999.

#### Fourth, the resulting gasoline volume is multiplied by 365 to identify an annualized value. In the case of leap years, the gasoline volume is multiplied by 366 to identify the annualized value.

### “Racing vehicle” means a vehicle that:

#### Is exclusively operated in conjunction with sanctioned racing events;

#### Exhibits racing features and modifications such that it is incapable of safe and practical street or highway use;

#### Is not licensed by the State of California Department of Motor Vehicles for operation on public streets or highways; and

#### Is never operated on public streets or highways.

### “Refiner” means any person who owns, leases, operates, controls or supervises a refinery.

### “Refinery” means a facility that produces liquid fuels by distilling petroleum.

### “Small refiner” means any refiner who owns or operates a refinery in California that:

#### Has and at all times had since January 1, 1978, a crude oil capacity of not more than 55,000 barrels per stream day;

#### Has not been at any time since September 1, 1988, owned or controlled by any refiner that at the same time owned or controlled refineries in California with a total combined crude oil capacity of more than 55,000 barrels per stream day; and

#### Has not been at any time since September 1, 1988, owned or controlled by any refiner that at the same time owned or controlled refineries in the United States with a total combined crude oil capacity of more than 137,500 barrels per stream day.

### “South Coast Area” means the counties of Los Angeles, Orange, Riverside, San Bernardino, and Ventura.

### “Stream day” means 24 consecutive hours of actual operation of a refinery.

### “Supply” means to provide or transfer a product to a physically separate facility, vehicle, or transportation system.

“Terminal” means a facility to store, blend, and distribute gasoline.

Note: Authority cited: Sections 39600, 39601, 43013, 43013.1, 43018 and 43101, Health and Safety Code. Reference: Sections 39000, 39001, 39002, 39003, 39010, 39048, 39500, 39515, 39516, 41511, 43000, 43013, 43013.1, 43016, 43018 and 43101, Health and Safety Code; and Western Oil and Gas Ass'n. v. Orange County Air Pollution Control District, 14 Cal.3d 411, 121 Cal. Rptr. 249 (1975).

# 2261. Applicability of Standards; Additional Standards

## Applicability of the CaRFG Phase 3 Standards.

#### The CaRFG Phase 3 cap limit standards set forth in section 2262, and the CaRFG Phase 3 cap limit compliance requirements in 2262.3(a), 2262.4(a), and 2262.5(a) and (b), shall apply starting December 31, 2003. The remaining CaRFG Phase 3 standards and compliance requirements contained in this subarticle shall apply to all sales, supplies, or offers of California gasoline occurring on or after December 31, 2003.

## California gasoline sold or supplied on or after March 1, 1996, is also subject to section 2253.4 (Lead/Phosphorus in Gasoline), section 2254 (Manganese Additive Content), and section 2257 (Required Additives in Gasoline).

## The standards contained in this subarticle shall not apply to a sale, offer for sale, or supply of California gasoline to a refiner if: (1) the refiner further processes the gasoline at the refiner's refinery prior to any subsequent sale, offer for sale, or supply of the gasoline, and (2) in the case of standards applicable only to producers or importers, the refiner to whom the gasoline is sold or supplied is the producer of the gasoline pursuant to section 2260.

## The prohibitions in sections 2262.3(b) and (c), 2262.4(b), and 2262.5(c) shall not apply to gasoline which a producer or importer demonstrates was neither produced nor imported by the producer or importer.

## This subarticle 2, section 2253.4 (Lead/Phosphorus in Gasoline), section 2254 (Manganese Additive Content), and section 2257 (Required Additives in Gasoline) shall not apply to gasoline where the person selling, offering or supplying the gasoline demonstrates as an affirmative defense that the person has taken reasonably prudent precautions to assure that the gasoline is used only in racing vehicles.

Note: Authority cited: Sections 39600, 39601, 43013, 43013.1, 43018 and 43101, Health and Safety Code. Reference: Sections 39000, 39001, 39002, 39003, 39010, 39500, 39515, 39516, 41511, 43000, 43013, 43013.1, 43016, 43018, 43101 and 43830.8, Health and Safety Code; and Western Oil and Gas Ass'n. v. Orange County Air Pollution Control District, 14 Cal.3d 411, 121 Cal. Rptr. 249 (1975).

# 2262. The California Reformulated Gasoline Phase 3 Standards.

The CaRFG Phase 3 standards are set forth in the following table. For all properties but Reid vapor pressure (cap limit only) and oxygen content, the value of the regulated property must be less than or equal to the specified limit. With respect to the Reid vapor pressure cap limit and the oxygen content flat and cap limit, the limits are expressed as a range, and the Reid vapor pressure and oxygen content must be less than or equal to the upper limit, and more than or equal to the lower limit.

A small refiner may comply with the small refiner CaRFG Phase 3 standards, in place of the CaRFG Phase 3 standards in this section, in accordance with section 2272.

The California Reformulated Gasoline Phase 3 Standards

| Property | Flat Limits |  | Averaging Limits |  | Cap Limits |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reid Vapor Pressure1 (pounds per square inch) |  | 7.00 or 6.902 |  | Not Applicable |  | 6.40 - 7.20 |
| Sulfur Content (parts per million by weight) |  | 20 |  | 15 |  |  |
|  |  |  |  |  |  | 20 |
| Benzene Content (percent by volume) |  | 0.80 |  | 0.70 |  | 1.10 |
| Aromatics Content (percent by volume) |  | 25.0 |  | 22.0 |  | 35.0 |
| Olefins Content (percent by volume) |  | 6.0 |  | 4.0 |  | 10.0 |
| T50 (degrees Fahrenheit) |  | 213 |  | 203 |  | 220 |
| T90 (degrees Fahrenheit) |  | 305 |  | 295 |  | 330 |
| Oxygen Content (percent by weight) E104 |  | 1.8 - 2.2 |  | Not Applicable |  | 1.83- 3.7 |
|  |  |  |  |  |  | 03 - 3.7 |
| Oxygen Content (percent by weight) E155 |  | 2.3 – 3.5 |  | Not Applicable |  | 3.8 – 6.0 |

1The Reid vapor pressure (RVP) standards apply only during the warmer weather months identified in section 2262.4.

2 The 6.90 pounds per square inch (psi) flat limit applies when a producer or importer is using the CaRFG Phase 3 Predictive Model to certify a final blend not containing ethanol. Otherwise, the 7.0 psi limit applies.

3 The 1.8 percent by weight minimum oxygen content cap only applies during specified winter months in the areas identified in section 2262.5(a).

4  E10 (ethanol up to 10.0% by volume) must have an oxygen content weight % value within the range specified in this table for E10.

5  E15 (ethanol from 10.1 to 15% by volume) must have an oxygen content weight % value within the range specified in this table for E15.

Note: Authority cited: Sections 39600, 39601, 43013, 43013.1, 43018, 43101 and 43830, Health and Safety Code. Reference: Sections 39000, 39001, 39002, 39003, 39010, 39500, 39515, 39516, 41511, 43000, 43013, 43013.1, 43016, 43018, 43101, 43830 and 43830.8, Health and Safety Code; and Western Oil and Gas Ass'n v. Orange County Air Pollution Control District, 14 Cal.3d 411, 121 Cal. Rptr. 249 (1975).

*    *    *    *

# 2262.3. Compliance with CaRFG Phase 3 Standards for Sulfur, Benzene, Aromatic Hydrocarbons, Olefins, T50 and T90.

## Compliance with cap limits. No person shall sell, offer for sale, supply, offer for supply, or transport California gasoline which exceeds an applicable cap limit for sulfur, benzene, aromatic hydrocarbons, olefins, T50 or T90 set forth in section 2262.

## Compliance by producers and importers with the flat limits. No producer or importer shall sell, offer for sale, supply, or offer for supply from its production facility or import facility California gasoline which exceeds an applicable flat limit for the properties of sulfur, benzene, aromatic hydrocarbons, olefins, T50, or T90 set forth in section 2262, unless the gasoline (1) is subject to the averaging compliance option for the property in accordance with section 2264.2(a), (2) has been reported as a Predictive Model alternative gasoline formulation pursuant to section 2265(a), or (3) has been reported as a Predictive Model emissions offsetting formulation pursuant to section 2265.1 (applicable only to producers and importers that produce gasoline). An importer that does not produce gasoline shall not sell, offer for sale, supply, or offer for supply California gasoline if the gasoline creates emissions associated with permeation.

## Optional compliance by producers and importers with the averaging limits. No producer or importer shall sell, offer for sale, supply, or offer for supply from its production facility or import facility California gasoline which is subject to the averaging compliance option for the properties of sulfur, benzene, aromatic hydrocarbons, olefins, T50 or T90 in accordance with section 2264.2(a) if any of the following occurs. An importer that does not produce gasoline shall not sell, offer for sale, supply, or offer for supply California gasoline if the gasoline creates emissions associated with permeation.

### The gasoline exceeds the applicable averaging limit for the property set forth in section 2262 and no designated alternative limit for the property has been established for the gasoline in accordance with the requirements of section 2264(a); or

### A designated alternative limit for the property has been established for the gasoline in accordance with the requirements of section 2264(a), and the gasoline exceeds the designated alternative limit for that property; or

### Where the designated alternative limit exceeds the averaging limit for the property, the exceedance is not fully offset in accordance with section 2264(c).

## Optional compliance by producers or importers that produce gasoline with the Predictive Model emissions offsetting compliance option. No producer or importer that produces gasoline shall sell, offer for sale, supply, or offer for supply from its production facility or import facility a final blend of California gasoline as a Predictive Model emissions offsetting formulation subject to the designated emissions offsetting limits if any of the following occurs:

### The gasoline exceeds any of the designated emissions offsetting limits for sulfur, benzene, aromatic hydrocarbons, olefins, oxygen, T50, or T90 for the final blend; or

### The gasoline exceeds the designated emissions offsetting limit for the final blend for RVP during any of the regulatory control periods in section 2262.4(b)(2).

### The excess emissions of oxides of nitrogen, total ozone forming potential, or the potency-weighted toxics associated with excess sulfur in a final blend of gasoline is not fully and timely offset in accordance with section 2265.1(c).

### The gasoline required to be offset and the offsetting gasoline are sold, offered, or supplied in different regions where the gasoline is normally offered for sale.

Note: Authority cited: Sections 39600, 39601, 43013, 43013.1, 43018 and 43101, Health and Safety Code. Reference: Sections 39000, 39001, 39002, 39003, 39010, 39500, 39515, 39516, 41511, 43000, 43013, 43013.1, 43016, 43018, 43101 and 43830.8, Health and Safety Code; and Western Oil and Gas Ass'n. v. Orange County Air Pollution Control District, 14 Cal.3d 411, 121 Cal. Rptr. 249 (1975).

# 2262.4. Compliance with the CaRFG Phase 3 Standards for Reid Vapor Pressure.

## Compliance with the cap limits for Reid vapor pressure.

### No person shall sell, offer for sale, supply, offer for supply, or transport California gasoline which exceeds the applicable cap limit for Reid vapor pressure within each of the air basins during the regulatory period set forth in section (a)(2).

### Regulatory Control Periods.

#### April 1 through October 31:

South Coast Air Basin and Ventura County

San Diego Air Basin

Mojave Desert Air Basin

Salton Sea Air Basin

#### May 1 through September 30:

Great Basin Valley Air Basin

#### May 1 through October 31:

San Francisco Bay Area Air Basin

San Joaquin Valley Air Basin

Sacramento Valley Air Basin

Mountain Counties Air Basin

Lake Tahoe Air Basin

#### June 1 through September 30:

North Coast Air Basin

Lake County Air Basin

Northeast Plateau Air Basin

#### June 1 through October 31:

North Central Coast Air Basin

South Central Coast Air Basin (Excluding Ventura County)

## Compliance by producers and importers with the flat limit for Reid vapor pressure.

### Reid vapor pressure standard for producers and importers.

#### In an air basin during the regulatory control periods specified in section (b)(2), no producer or importer shall sell, offer for sale, supply, or offer for supply from its production facility or import facility California gasoline which has a Reid vapor pressure exceeding the applicable flat limit set forth in section 2262 unless the gasoline has been reported as either a Predictive Model alternative gasoline formulation pursuant to section 2265(a) or a Predictive Model emissions offsetting formulation pursuant to section 2265.1 (applicable only to producers and importers that produce gasoline) using the evaporative emissions model element of the CaRFG Phase 3 Predictive Model.

#### In an air basin during the regulatory control periods specified in section (b)(2), no producer or importer shall sell, offer for sale, supply, or offer for supply from its production facility or import facility California gasoline which has been reported either as a Predictive Model alternative gasoline formulation pursuant to section 2265(a) or a Predictive Model emissions offsetting formulation pursuant to section 2265.1 (applicable only to producers and importers that produce gasoline) using the evaporative emissions model element of the CaRFG Phase 3 Predictive Model if the gasoline has a Reid vapor pressure exceeding the Predictive Model flat limit for Reid vapor pressure in the identified Predictive Model alternative specifications or the designated emissions offsetting limits, as applicable.

### Regulatory control periods for production and import facilities.

##### March 1 through October 31:

South Coast Air Basin and Ventura County

San Diego Air Basin

Mojave Desert Air Basin

Salton Sea Air Basin

#### April 1 through September 30:

Great Basin Valley Air Basin

#### April 1 through October 31:

San Francisco Bay Area Air Basin

San Joaquin Valley Air Basin

Sacramento Valley Air Basin

Mountain Counties Air Basin

Lake Tahoe Air Basin

#### May 1 through September 30:

North Coast Air Basin

Lake County Air Basin

Northeast Plateau Air Basin

#### May 1 through October 31:

North Central Coast Air Basin

South Central Coast Air Basin (Excluding Ventura County)

## Applicability.

### Section (a)(1) shall not apply to a transaction occurring in an air basin during a regulatory control period in section (a)(2) where the person selling, supplying, or offering the gasoline demonstrates as an affirmative defense that, prior to the transaction, he or she has taken reasonably prudent precautions to assure that the gasoline will be delivered to a retail service station or bulk purchaser-consumer's fueling facility when the station or facility is not subject to a regulatory control period in section (a)(2).

### Section (b) shall not apply to a transaction occurring in an air basin during the applicable regulatory control period for producers and importers where the person selling, supplying, offering or transporting the gasoline demonstrates as an affirmative defense that, prior to the transaction, he or she has taken reasonably prudent precautions to assure that the gasoline will be delivered to a retail service station or bulk purchaser-consumer's fueling facility located in an air basin not then subject to the regulatory control period for producers and importers set forth in section (b)(2).

### Section (a)(1) shall not apply to a transaction occurring in an air basin during the regulatory control period where the transaction involves the transfer of gasoline from a stationary storage tank to a motor vehicle fuel tank and the person selling, supplying, or offering the gasoline demonstrates as an affirmative defense that the last delivery of gasoline to the stationary storage tank occurred more than fourteen days before the start of the regulatory control period.

### Gasoline that is produced in California, and is then transported to the South Coast Air Basin, Ventura County, or the San Diego Air Basin by marine vessel shall be subject to the regulatory control periods for production and import facilities identified in section 2262.4(b)(2)(A).

Note: Authority cited: Sections 39600, 39601, 43013, 43013.1, 43018 and 43101, Health and Safety Code. Reference: Sections 39000, 39001, 39002, 39003, 39010, 39500, 39515, 39516, 41511, 43000, 43013, 43013.1, 43016, 43018, 43101, 43830 and 43830.8, Health and Safety Code; and Western Oil and Gas Ass'n. v. Orange County Air Pollution Control District, 14 Cal.3d 411, 121 Cal. Rptr. 249 (1975).

# 2262.5. Compliance with the Standards for Oxygen Content.

## Compliance with the minimum oxygen content cap limit standard in specified areas in the wintertime.

### Within the areas and periods set forth in section (a)(2), no person shall sell, offer for sale, supply, offer for supply, or transport California gasoline unless it has an oxygen content of not less than the minimum oxygen content cap limit in section 2262.

#### November 1 through February 29:

South Coast Area

Imperial County

## Compliance with the maximum oxygen content cap limit standard. No person shall sell, offer for sale, supply, or transport California gasoline which has an oxygen content exceeding the maximum oxygen content cap limit in section 2262, or which has an ethanol content exceeding 15 percent by volume.

## Compliance by producers and importers with the flat limits for oxygen content. No producer or importer shall sell, offer for sale, supply, or offer for supply from its production or import facility California gasoline which has an oxygen content less than flat limit for minimum oxygen content, or more than flat limit for maximum oxygen content, unless the gasoline has been (1) reported as a Predictive Model alternative gasoline formulation pursuant to section 2265(a), or as a Predictive Model emissions offsetting formulation pursuant to section 2265.1 (applicable only to producers and importers that produce gasoline), and (2) complies with the standards contained in sections (a) and (b).

## Application of prohibitions.

### Section (a) shall not apply to a transaction occurring in the areas and periods shown in (a)(2) where the person selling, supplying, or offering the gasoline demonstrates as an affirmative defense that, prior to the transaction, he or she has taken reasonably prudent precautions to assure that the gasoline will not be delivered to a retail service station or bulk purchaser-consumer's fueling facility in the areas and periods shown in (a)(2).

Note: Authority cited: Sections 39600, 39601, 43013, 43013.1, 43018 and 43101, Health and Safety Code. Reference: Sections 39000, 39001, 39002, 39003, 39010, 39500, 39515, 39516, 41511, 43000, 43013, 43013.1, 43016, 43018, 43101 and 43830.8, Health and Safety Code; and Western Oil and Gas Ass'n v. Orange County Air Pollution Control District, 14 Cal.3d 411, 121 Cal. Rptr. 249 (1975).

# 2262.6. Prohibition of MTBE and Oxygenates Other Than Ethanol in California Gasoline Starting December 31, 2003.

## Basic MTBE prohibitions.

### Starting December 31, 2003, no person shall sell, offer for sale, supply or offer for supply California gasoline which has been produced at a production facility in part by either (i) adding at the production facility any methyl tertiary-butyl ether (MTBE) in neat form to the California gasoline or to a blending component used in the gasoline; or (ii) using a blending component that contained greater than 0.60 volume percent MTBE when it was supplied to the production facility.

### No person shall sell, offer for sale, supply or offer for supply California gasoline which contains MTBE in concentrations greater than 0.05 volume percent starting July 1, 2007.

## Use of oxygenates other than ethanol or MTBE in California gasoline on or after December 31, 2003.

### Starting December 31, 2003, no person shall sell, offer for sale, supply or offer for supply California gasoline which has been produced at a production facility with the use of any oxygenate other than ethanol or MTBE unless a multimedia evaluation of use of the oxygenate in California gasoline has been conducted and the California Environmental Policy Council established by Public Resources Code section 71017 has determined that such use will not cause a significant adverse impact on the public health or the environment.

### Starting July 1, 2004, no person shall sell, offer for sale, supply or offer for supply California gasoline which contains a total of more than 0.06 weight percent oxygen collectively from all of the oxygenates identified in section (c)(4), other than oxygenates not prohibited by section (c)(1).

### Covered oxygenates. Oxygen from the following oxygenates is covered by the prohibitions in section (b)(1) and (2):

Methanol

Isopropanol

n-Propanol

n-Butanol

iso-Butanol

sec-Butanol

tert-Butanol

Tert-pentanol (tert-amylalcohol)

Ethyl tert-butylether (ETBE)

Diisopropylether (DIPE)

Tert-amylmethylether (TAME)

Note: Authority cited: Sections 39600, 39601, 43013, 43013.1, 43018 and 43101, Health and Safety Code. Reference: Sections 39000, 39001, 39002, 39003, 39010, 39500, 39515, 39516, 41511, 43000, 43013, 43013.1, 43016, 43018, 43101 and 43830.8, Health and Safety Code; and Western Oil and Gas Ass'n. v. Orange County Air Pollution Control District, 14 Cal.3d 411, 121 Cal. Rptr. 249 (1975).

# 2262.9 Requirements Regarding Denatured Ethanol Intended For Use as a Blend Component in California Gasoline.

## Standards.

### Standards for denatured ethanol. Starting December 31, 2003, no person shall sell, offer for sale, supply or offer for supply denatured ethanol intended for blending with CARBOB or California gasoline that fails to comply with any of the following standards:

#### Standards for properties regulated by the CaRFG Phase 3 standards.

##### A sulfur content not exceeding 10 parts per million;

##### A benzene content not exceeding 0.06 percent by volume; and

##### An olefins content not exceeding 0.5 percent by volume; and

##### An aromatic hydrocarbon content not exceeding 1.7 percent by volume.

#### Standards based on ASTM D 4806-99. All test methods and standards identified in the title and the table below are incorporated herein by reference.

*    *    *    *

## Test Methods.

### In determining compliance with the denatured ethanol standards in section (a)(1)(A):

#### The sulfur content of denatured ethanol shall be determined by ASTM D 5453-93, which is incorporated herein by reference.

#### The aromatic hydrocarbon, benzene and olefins content of denatured ethanol shall be determined by sampling the denaturant and using the methods specified in section 2263 to determine the content of those compounds in the denaturant. The result will then be multiplied by 0.0500, except that where it is demonstrated that the denatured ethanol contains less than 5.00 percent denaturant, the result will be multiplied by the decimal fraction representing the percent denaturant; or

#### Starting February 16, 2014, the aromatic hydrocarbon and benzene content of denatured ethanol shall be determined by ASTM D7576-10 (2010), which is incorporated herein by reference. Starting February 16, 2014, the olefin content of denatured ethanol shall be determined by ASTM D7347-07e1 (2007), which is incorporated herein by reference.

#### In the event of any discrepancy between results obtained by using sections (b)(1)(B) and (b)(1)(C), the results obtained by using section (b)(1)(C) shall take precedence.

### In determining compliance with the denaturant standards in section (a)(3), the aromatic hydrocarbon, benzene and olefins content of the denaturant shall be determined by the methods specified in section 2263 for determining the content of those compounds in gasoline.

## Documentation required for the transfer of denatured ethanol intended for use as a blend component in California gasoline.

*    *    *    *

Note: Authority cited: Sections 39600, 39601, 43013, 43013.1, 43018 and 43101, Health and Safety Code. Reference: Sections 39000, 39001, 39002, 39003, 39010, 39500, 39515, 39516, 41511, 43000, 43013, 43013.1, 43016, 43018, 43101 and 43830.8, Health and Safety Code; and Western Oil and Gas Ass'n. v. Orange County Air Pollution Control District, 14 Cal.3d 411, 121 Cal. Rptr. 249 (1975).

# 2263. Sampling Procedures and Test Methods.

*    *    *    *

## Test Methods.

### In determining compliance with the gasoline standards set forth in this subarticle 2, including those in the sections identified in Table 1, the test methods presented in Table 1 shall be used. All identified test methods are incorporated herein by reference.

Table 1

| Section | Gasoline Specification | Test Methoda |
| --- | --- | --- |
| 2262 | Reid Vapor Pressure | ASTM D 323-58b or 13 C.C.R. Section 2297 |
| 2262 | Sulfur Content | ASTM D 5453-93 (1993) |
| 2262 | Benzene Content | ASTM D 5580-02 (2007) |
| 2262 | Olefin Content | ASTM D 6550-10 (2010)k,l,m |
| 2262 | Oxygen Content | ASTM D 4815-09 (2009) |
| 2262 | T90 and T50 | ASTM D 86-99aε1 |
| 2262 | Aromatic Hydrocarbon Content | ASTM D5580-02 (2007) |
| 2262.5(b) | Ethanol Content | ASTM D 4815-09 (2009) |
| 2262.6 | MTBE Content | ASTM D 7754-11 (2011) |
| 2262.6(b) | Oxygen from oxygenates identified in section 2262.6(b)(3) | ASTM D 7754-11 (2011) |

*    *    *    *

Note: Authority cited: Sections 39600, 39601, 43013, 43013.1, 43018 and 43101, Health and Safety Code. Reference: Sections 39000, 39001, 39002, 39003, 39010, 39500, 39515, 39516, 41511, 43000, 43013, 43013.1, 43016, 43018, 43101, Health and Safety Code; and Western Oil and Gas Ass'n. v. Orange County Air Pollution Control District, 14 Cal.3d 411, 121 Cal. Rptr. 249 (1975).

# 2263.7. Multiple Notification Requirements.

Where a producer or importer is subject to multiple notification requirements pursuant to sections 2264(a)(2)(A), 2265.1 (applicable only to producers and importers that produce gasoline), 2264.2(a)(2), 2264.2(b)(2), 2265(a)(2) or 2266.5(b), the producer or importer shall combine the notifications to the extent practicable.

Note: Authority cited: Sections 39600, 39601, 43013, 43013.1, 43018 and 43101, Health and Safety Code. Reference: Sections 39000, 39001, 39002, 39003, 39010, 39500, 39515, 39516, 41511, 43000, 43013, 43013.1, 43016, 43018 and 43101, Health and Safety Code; and Western Oil and Gas Ass'n. v. Orange County Air Pollution Control District, 14 Cal.3d 411, 121 Cal. Rptr. 249 (1975).

# 2264. Designated Alternative Limits.

*    *    *    *

## Designated alternative limits for Predictive Model alternative gasoline formulations. The producer or importer of a final blend of California gasoline that is subject to the Predictive Model averaging compliance option for one or more properties may assign a designated alternative limit to the final blend by satisfying the notification requirements of section (a). The producer or importer of such a final blend shall be subject to all of the provisions of this section, except that, with respect to that final blend, the Predictive Model averaging limit (if any) for each property subject to the Predictive Model averaging compliance option shall replace any reference in this section to the averaging limit specified in section 2262.

Note: Authority cited: Sections 39600, 39601, 43013, 43013.1, 43018 and 43101, Health and Safety Code. Reference: Sections 39000, 39001, 39002, 39003, 39010, 39500, 39515, 39516, 41511, 43000, 43013, 43013.1, 43016, 43018 and 43101, Health and Safety Code; and Western Oil and Gas Ass'n. v. Orange County Air Pollution Control District, 14 Cal.3d 411, 121 Cal. Rptr. 249 (1975).

# 2264.2. Election of Applicable Limit for Gasoline Supplied From a Production or Import Facility.

## Election of the averaging compliance option.

### A producer or importer selling or supplying a final blend of gasoline from its production or import facility may elect pursuant to this section (a) to have the final blend subject to the averaging compliance option for one or more of the following properties: sulfur, benzene, olefins, aromatic hydrocarbons, T90 or T50. Once a producer or importer has made such an election for a gasoline property, all final blends subsequently sold or supplied from the production or import facility shall be subject to the averaging compliance option for that property until the producer or importer either (A) elects in accordance with section (b) to have a final blend at the facility subject to the flat limit compliance option for that property, or (B) elects in accordance with section 2265(a) to sell or supply a final blend at the facility as a Predictive Model alternative gasoline formulation.

### In order to elect to have a final blend subject to the averaging option for a gasoline property, the producer or importer shall notify the executive officer of such election and of the information identified in section 2264(a)(2)(A), within the time limits set forth in section 2264(a)(2)(A) and subject to section 2264(a)(3) and (4).

### A producer or importer may not elect to sell, offer, or supply from its production facility or import facility a final blend of California gasoline subject to the averaging compliance option in section 2264 if the producer or importer is subject to any outstanding requirements to provide Predictive Model emissions offsets pursuant to section 2265.1(c).

## Election of flat limit compliance option.

### A producer or importer selling or supplying a final blend of gasoline from its production or import facility may elect to have the final blend subject to the flat limit compliance option in accordance with this section (b). No such election may be made if there are outstanding requirements to provide offsets for the gasoline property at the facility pursuant to section 2264(c).

### In order to elect to have a final blend subject to the flat limit compliance option for a gasoline property, the producer or importer shall notify the executive officer of such election and of the blend identity and the location of the final blend, within the time limits set forth in section 2264(a)(2)(A) and subject to section 2264(a)(3) and (4).

### Once a producer or importer has made an election under this section (b) with respect to a gasoline property, all final blends subsequently sold or supplied from the production or import facility shall be subject to the flat limit compliance option for that property until the producer or importer either (A) elects in accordance with section (a) to have a final blend at the facility subject to the averaging compliance option for that property, or (B) elects in accordance with section 2265(a) to sell or supply a final blend at the facility as a Predictive Model alternative gasoline formulation, or (C) elects in accordance with section (d) to sell or supply a final blend at the facility as a Predictive Model emissions offsetting formulation (applicable only to producers and importers that produce gasoline).

### Once a producer or importer has made an election under this section (b) with respect to a gasoline property of a final blend at a production or import facility, the producer or importer may not use any previously assigned designated alternative limit for that property to provide offsets pursuant to section 2264(c) for any final blend sold or supplied from the production or import facility subsequently to the election.

### A producer or importer may not elect to sell, offer, or supply from its production facility or import facility a final blend of California gasoline subject to the flat limit compliance option if the producer or importer is subject to any outstanding requirements to provide Predictive Model emissions offsets pursuant to section 2265.1(c).

## Inapplicability to elections for Predictive Model alternative gasoline formulations. Any election for a final blend to be subject to a Predictive Model averaging compliance option or a Predictive Model flat limit compliance option shall be made in accordance with section 2265 rather than this section.

## Election of the Predictive Model emissions offsetting compliance option.

### Applicability. A producer or importer that produces gasoline and is selling, offering, or supplying a final blend of gasoline from its production facility or import facility may elect pursuant to this section (d) to have the final blend subject to the Predictive Model emissions offsetting compliance option when all of the following conditions are satisfied:

#### With regard to a batch of gasoline that does not meet the criteria for approval in the applicable Predictive Model Procedures, immediately prior to producing that batch, the producer or importer has reported its gasoline as a Predictive Model alternative gasoline formulation pursuant to section 2265(a),

#### The actual sulfur content in the Predictive Model alternative gasoline formulation exceeds the Predictive Model alternative specification,

#### But for the elevated sulfur content, the Predictive Model alternative specifications would have met the criteria for approval in the applicable Predictive Model Procedures,

#### The gasoline reported as a Predictive Model alternative gasoline formulation has a percent change in emissions value either for oxides of nitrogen, total ozone forming potential, or potency-weighted toxics that results in a final blend deficit, and

#### The producer or importer is not subject to any outstanding requirements to provide offsets at the same production facility or import facility pursuant to section 2264(c).

### Once a producer or importer has made such an election for a final blend of gasoline, all final blends subsequently sold or supplied from the production facility or import facility, whether associated with a final blend deficit or a final blend credit, shall be subject to the Predictive Model emissions offsetting compliance option until the producer or importer either (A) elects in accordance with section (a) to have a final blend at the facility subject to the averaging compliance option, or (B) elects in accordance with section (b) to have a final blend at the facility subject to the flat limit compliance option for all fuel properties, or (C) elects in accordance with section 2265(a) to sell or supply a final blend at the facility as a Predictive Model alternative gasoline formulation.

### In order to elect to have a final blend subject to the Predictive Model emissions offsetting compliance option for a final blend, the producer or importer shall notify the Executive Officer of such election and of the information identified in section 2265.1(a)(2)(A), within the time limits set forth in section 2265.1(a)(2)(A) and subject to section 2265.1(a)(3) and (4).

### Once a producer or importer has made an election under this section (d) with respect to the Predictive Model emissions offsetting compliance option, the producer or importer may not use any previously assigned designated alternative limit for any fuel property to provide offsets pursuant to section 2264(c) for any final blend sold or supplied from the production facility or import facility subsequent to the election.

Note: Authority cited: Sections 39600, 39601, 43013, 43013.1, 43018 and 43101, Health and Safety Code. Reference: Sections 39000, 39001, 39002, 39003, 39010, 39500, 39515, 39516, 41511, 43000, 43013, 43013.1, 43016, 43018 and 43101, Health and Safety Code; and Western Oil and Gas Ass'n. v. Orange County Air Pollution Control District, 14 Cal.3d 411, 121 Cal. Rptr. 249 (1975).

# 2265. Gasoline Subject to Predictive Model Alternative Specifications Based on California Predictive Model.

## Election to sell or supply a final blend as a Predictive Model alternative gasoline formulation.

### In order to sell or supply from its production facility or import facility a final blend of California gasoline as a Predictive Model alternative gasoline formulation subject to Predictive Model alternative specifications, a producer or importer shall satisfy the requirements of this section (a).

#### Evaluation of the Candidate Predictive Model Alternative Specifications.

##### The producer or importer shall evaluate the candidate Predictive Model alternative specifications in accordance with the applicable “Predictive Model Procedures” documents incorporated by reference below.

##### Gasoline with up to 10.0 Percent Ethanol by Volume . For a final blend starting to be sold or supplied from the production or import facility , the producer or importer shall evaluate the candidate Predictive Model alternative specifications for gasoline subject to the CaRFG Phase 3 standards in accordance with the Air Resources Board's “California Procedures for Evaluating Alternative Specifications for Phase 3 Reformulated Gasoline Using the California Predictive Model,” last amended August 24, 2012, which is incorporated herein by reference.

##### Gasoline with 10.1 to 15 Percent Ethanol by Volume. A producer or importer may blend additional ethanol into E10 compliant with subsection 2265(a)(2)(A)2 immediately above to produce E15.

#### Notification to the Executive Officer. If the Predictive Model alternative specifications being evaluated meet the criteria for approval in the applicable Predictive Model Procedures, the producer or importer shall notify the Executive Officer of:

##### The identity and location of the final blend;

##### the Predictive Model alternative specifications that will apply to the final blend, including for each specification whether it applies as a Predictive Model flat limit or a Predictive Model averaging limit;

##### the numerical values for percent change in emissions for oxides of nitrogen, total ozone forming potential, and potency-weighted toxic air contaminants as determined in accordance with the applicable Predictive Model Procedures;

##### the grade of gasoline of the final blend;

##### the location of the final blend with sufficient specificity to locate and sample the gasoline. This shall include, but is not limited to, the name of the facility, address, and identification of the storage tank.

The notification shall be received by the Executive Officer before the start of physical transfer of the gasoline from the production or import facility, and in no case less than 12 hours before the producer or importer either completes physical transfer or commingles the final blend.

### Once a producer or importer has notified the executive officer pursuant to this section (a) that a final blend of California gasoline is being sold or supplied from a production or import facility as a Predictive Model alternative gasoline formulation, all final blends of California gasoline subsequently sold or supplied from that production or import facility shall be subject to the same Predictive Model alternative specifications until the producer or importer either (A) designates a final blend at that facility as a Predictive Model alternative gasoline formulation subject to different Predictive Model alternative specifications, (B) elects in accordance with section 2264.2 to have a final blend at that facility subject to flat limit compliance options and/or averaging compliance options, or (C) elects in accordance with section 2264.2(d) to sell or supply a final blend at that facility as a Predictive Model emissions offsetting formulation (applicable only to producers and importers that produce gasoline).

### The executive officer may enter into a written protocol with any individual producer or importer for the purposes of specifying how the requirements in section (a)(2) shall be applied to the producer's or importer's particular operations, as long as the executive officer reasonably determines that application of the regulatory requirements under the protocol is not less stringent or enforceable than application of the express terms of section (a)(2). Any such protocol shall include the producer's or importer's agreement to be bound by the terms of the protocol.

### If, through no intentional or negligent conduct, a producer or importer cannot report within the time period specified in section (a)(2) above, the producer or importer may notify the executive officer of the required data as soon as reasonably possible and may provide a written explanation of the cause of the delay in reporting. If, based on the written explanation and the surrounding circumstances, the executive officer determines that the conditions of this section (a)(5) have been met, timely notification shall be deemed to have occurred.

## Prohibited activities regarding Predictive Model alternative gasoline formulations.

### No producer or importer shall sell, offer for sale, supply, or offer for supply from its production or import facility California gasoline which is reported pursuant to section (a) as a Predictive Model alternative gasoline formulation subject to Predictive Model alternative specifications if any of the following occur:

#### The identified Predictive Model alternative specifications do not meet the criteria for approval in the applicable Predictive Model Procedures; or

#### The producer was prohibited by section (c) from electing to sell or supply the gasoline as a Predictive Model alternative gasoline formulation; or

#### The gasoline fails to conform with any Predictive Model flat limit in the identified Predictive Model alternative specifications (see section 2262.4(b) in the case of specifications for Reid vapor pressure); or

#### With respect to any property for which the producer or importer has identified a Predictive Model averaging limit.

##### the gasoline exceeds the applicable Predictive Model average limit, and no designated alternative limit for the property has been established for the gasoline in accordance with section 2264(a); or

##### a designated alternative limit for the property has been established for the gasoline in accordance with section 2264(a), and either of the following occur:

###### The gasoline exceeds the designated alternative limit for the property, or

###### Where the designated alternative limit for the property exceeds the Predictive Model averaging limit, the exceedance is not fully offset in accordance with the applicable provisions in section 2264(c).

### Where a producer or importer has elected to sell or supply a final blend of California gasoline as a Predictive Model alternative gasoline formulation in accordance with this section, the final blend shall not be subject to section 2262.3(b) and (c), section 2262.4(b), and section 2262.5(c).

## Restrictions associated with elections to sell or supply final blends as Predictive Model alternative gasoline formulations.

### A producer or importer may not elect to sell or supply from its production or import facility a final blend of California gasoline as a Predictive Model alternative gasoline formulation if the producer or importer is subject to any outstanding requirements to provide offsets at the same production or import facility pursuant to section 2264(c).

### Once a producer or importer has elected to sell or supply from its production or import facility a final blend of California gasoline as a Predictive Model alternative gasoline formulation subject to a Predictive Model averaging compliance option for one or more properties, the producer or importer may not elect any other compliance option, including another Predictive Model alternative gasoline formulation, if there are outstanding requirements to provide offsets for such property or properties pursuant to section 2264(c). However, this section (c)(2) shall not preclude a producer or importer under the circumstances described above from electing another Predictive Model alternative gasoline formulation where:

#### the only changes are that either:

##### Predictive Model flat limits for one or more properties are changed to Predictive Model averaging limits, or

##### a single Predictive Model averaging limit for which there are no outstanding requirements to provide offsets is changed to a Predictive Model flat limit, and

#### there are no changes to the Predictive Model alternative specifications for the remaining properties, and

#### the new Predictive Model alternative formulation meets the criteria for approval in the applicable Predictive Model Procedures.

### Once a producer or importer has elected to sell or supply from its production or import facility a final blend of California gasoline as a Predictive Model alternative gasoline formulation, the producer or importer may not use any previously assigned designated alternative limit for a property to provide offsets pursuant to section 2264(c) for any final blend sold or supplied from the production or import facility subsequent to the election.

### A producer or importer may not elect to sell or supply from its production facility or import facility a final blend of California gasoline as a Predictive Model alternative gasoline formulation if the producer or importer is subject to any outstanding requirements to provide offsets pursuant to section 2265.1(c).

Note: Authority cited: Sections 39600, 39601, 43013, 43013.1, 43018 and 43101, Health and Safety Code. Reference: Sections 39000, 39001, 39002, 39003, 39010, 39500, 39515, 39516, 41511, 43000, 43013, 43013.1, 43016, 43018 and 43101 , Health and Safety Code; and Western Oil and Gas Ass'n v. Orange County Air Pollution Control District, 14 Cal.3d 411, 121 Cal. Rptr. 249 (1975).

# 2265.1 Offsetting Emissions Associated with Higher Sulfur Levels.

## Assignment of designated emissions offsetting limits and percent change in emissions values for batches of gasoline for which the emissions associated with higher sulfur levels are being offset.

### A producer or an importer that produces gasoline that has elected to be subject to the Predictive Model emissions offsetting compliance option must assign a designated emissions offsetting limit for the sulfur, benzene, olefin, aromatic hydrocarbon, and oxygen (maximum and minimum) contents, and for the T90, T50, and RVP (during the RVP regulatory control period in section 2262.4(b)(2)) for each final blend of California gasoline produced by the producer or the importer and satisfying the notification requirements in this section (a). In no case shall a designated emissions offsetting limit be less than the sulfur, benzene, olefin, or aromatic hydrocarbon contents, T90, T50, or RVP, or less than the maximum oxygen content or greater than the minimum oxygen content of the final blend shown by the sample and test conducted pursuant to section 2270. For each final blend, the producer or the importer that produces gasoline shall also assign the percent change in emissions values, as they pertain to the Predictive Model emissions offsetting compliance option, for oxides of nitrogen, total ozone forming potential, and potency-weighted toxics.

### Notification of final blends associated with a final blend deficit

#### Except as otherwise provided, for each final blend that has a percent change in emissions value greater than 0.04 percent for oxides of nitrogen, total ozone forming potential, or potency-weighted toxics (referred to as the deficit final blend), the producer or the importer that produces gasoline shall notify the executive officer in writing, for receipt by the executive officer before the start of physical transfer of the gasoline from the production facility or the import facility, and in no case less than 12 hours before the producer or the importer that produces gasoline either completes physical transfer or commingles the final blend, with the following information:

##### Justification for using the Predictive Model emissions offsetting compliance option, including but not limited to, an explanation for the elevated sulfur level in the final blend and why the batch will not be re-blended or a different predictive model formulation will not be developed,

##### The targeted Predictive Model alternative specifications that the producer or the importer that produces gasoline was intending to produce and which would have resulted in a passing Predictive Model formulation but for the actual sulfur content of the blend,

##### The percent change in emissions values, as they pertain to the Predictive Model emissions offsetting compliance option, for oxides of nitrogen, total ozone forming potential, and potency-weighted toxics for the targeted Predictive Model alternative specifications,

##### The company name, address, phone number, and contact information,

##### The production facility or import facility name, batch name, number, or other identification, the blend identity, grade of California gasoline, the location (with sufficient specificity to allow CARB inspectors to locate and sample the gasoline; this shall include, but is not limited to, the name of the facility, address, and identification of the tank), and other information that uniquely identifies the California gasoline subject to the Predictive Model emissions offsetting compliance option,

##### The estimated volume (in barrels),

##### The designated emissions offsetting limit for RVP, sulfur content, benzene content, aromatics content, olefins content, T50, T90, and oxygen content for the final blend,

##### The percent change in emissions values, as they pertain to the Predictive Model emissions offsetting compliance option, for oxides of nitrogen, total ozone forming potential, and potency-weighted toxics for the final blend,

##### A statement, signed by a legal representative for the producer or the importer that produces gasoline that all information submitted with the notification is true and correct, and

##### Within 24 hours after the start of the physical transfer, the date and time of the start of physical transfer from the production facility or import facility.

#### A producer or importer that produces gasoline may report an actual volume that is less than the estimated volume, as long as notification of the actual volume is received by the executive officer no later than 48 hours after completion of the physical transfer of the final blend from the production facility or import facility. If notification of the actual volume is not timely received by the executive officer, the reported estimated volume shall be deemed the reported actual volume. If the actual volume is larger than initially estimated, the producer or the importer that produces gasoline shall revise the reported estimated volume by notifying the executive officer no later than 24 hours after completion of the physical transfer of the final blend from the production facility or import facility.

### Notification of final blends associated with a final blend credit.

#### For each final blend associated with a final blend credit, the producer or the importer that produces gasoline shall notify the Executive Officer in writing for receipt by the Executive Officer before the start of physical transfer of the gasoline from the production facility or the import facility, and in no case less than 12 hours before the producer or the importer that produces gasoline either completes physical transfer or commingles the final blend, with the following information:

##### The company name, address, phone number, and contact information,

##### The production facility or the import facility name, batch name, number, or other identification, the blend identity, grade of California gasoline, the location (with sufficient specificity to allow CARB inspectors to locate and sample the gasoline; this shall include, but is not limited to, the name of the facility, address, and identification of the tank), and other information that uniquely identifies the California gasoline associated with a final blend credit,

##### The estimated volume (in barrels),

##### The designated emissions offsetting limits for RVP, sulfur content, benzene content, aromatics content, olefins content, T50, T90, and oxygen content for the final blend,

##### The percent change in emissions values, as they pertain to the Predictive Model emissions offsetting compliance option, for oxides of nitrogen, total ozone forming potential, and potency-weighted toxics for the final blend,

##### A statement, signed by a legal representative for the producer or the importer that produces gasoline that all information submitted with the notification is true and correct, and

##### Within 24 hours after the completion of the physical transfer, the date and time of the completion of physical transfer from the production facility or the import facility.

#### A producer or importer that produces gasoline may report an actual volume that is more than the estimated volume, as long as notification of the actual volume is received by the executive officer no later than 48 hours after completion of the physical transfer of the final blend from the production facility or the import facility. If notification of the actual volume is not timely received by the executive officer, the reported estimated volume shall be deemed the reported actual volume. If the actual volume is less than initially estimated, the producer or the importer that produces gasoline shall revise the reported estimated volume by notifying the executive officer no later than 24 hours after completion of the physical transfer of the final blend from the production facility or import facility.

### If, through no intentional or negligent conduct, a producer or importer that produces gasoline cannot report within the time period specified in (a)(2) or (a)(3) above, the producer or importer that produces gasoline may notify the executive officer of the required data as soon as reasonably possible and may provide a written explanation of the cause of the delay in reporting. If, based on the written explanation and the surrounding circumstances, the executive officer determines that the conditions of this section (a)(4) have been met, timely notification shall be deemed to have occurred.

### The executive officer may enter into a written protocol with any individual producer or importer that produces gasoline for the purposes of specifying how the requirements in sections (a)(2) and (c) shall be applied to the producer's or importer's particular operations, as long as the executive officer reasonably determines that application of the regulatory requirements under the protocol is not less stringent or enforceable than application of the express terms of sections (a)(2) and (c). Any such protocol shall include the producer's or importer's agreement to be bound by the terms of the protocol.

## Additional prohibitions regarding gasoline with designated emissions offsetting limits and final blend credits or deficits.

### No producer or importer that produces gasoline shall sell, offer for sale, or supply California gasoline with a final blend deficit, where the total volume of the final blend sold, offered for sale, or supplied exceeds the volume reported to the executive officer pursuant to section (a).

### No producer or importer that produces gasoline shall sell, offer for sale or supply California gasoline with a final blend credit, where the total volume of the final blend sold, offered for sale, or supplied is less than the volume reported to the executive officer pursuant to section (a).

### Final blend credits shall not include offsets or other reductions that are otherwise required by any local, State, or federal rule, regulation, or statute, or that are achieved or estimated from California gasoline not produced or imported in the same air basin as the gasoline with a final blend deficit.

## Offsetting a final blend deficit. With respect to each final blend for which a producer or importer that produces gasoline has elected to be subject to the Predictive Model emissions offsetting compliance option, within 90 days after the start of physical transfer from a production facility or import facility of any final blend of California gasoline with a final blend deficit, the producer or importer shall complete physical transfer from the same production facility or import facility of California gasoline with a final blend credit in sufficient quantity and for the same emissions parameter (oxides of nitrogen, total ozone forming potential, or potency-weighted toxics) to fully offset the final blend deficit.

For example, within 90 days after the start of physical transfer from a production facility or import facility of 100 barrels of any final blend of California gasoline to which a producer or importer that produces gasoline has assigned a designated emissions offsetting limit which results in a 0.10 percent increase in oxides of nitrogen, the producer or importer that produces gasoline shall complete physical transfer from the same production facility or import facility of California gasoline in sufficient quantity and quality to offset the 6 deficit points for oxides of nitrogen. The final blend deficit is calculated as:

Final Blend Deficit = (0.10-0.04) x 100 = 6

## Automatic termination of the producer's or importer's use of the Predictive Model emissions offsetting compliance option.

When a producer or importer that produces gasoline has fully offset the final blend deficit, the producer's or importer's use of the Predictive Model emissions offsetting compliance option automatically terminates. Prior to selling, supplying, or offering California gasoline after the termination, the producer or importer that produces gasoline must elect to use the flat limits, designated alternative limits, or Predictive Model alternative specifications for its next final blend. The producer or importer that produces gasoline may not use any remaining final blend credits to provide offsets pursuant to section (c) for any final blend sold, offered, or supplied from the production facility or import facility subsequent to the election.

Note: Authority cited: Sections 39600, 39601, 43013, 43013.1, 43018 and 43101, Health and Safety Code. Reference: Sections 39000, 39001, 39002, 39003, 39010, 39500, 39515, 39516, 41511, 43000, 43013, 43013.1, 43016, 43018 and 43101, Health and Safety Code; and Western Oil and Gas Ass'n. v. Orange County Air Pollution Control District, 14 Cal.3d 411, 121 Cal. Rptr. 249 (1975).

### -

# 2266.5. Requirements Pertaining to California Reformulated Gasoline Blendstock for Oxygen Blending (CARBOB) and Downstream Blending.

## Application of the California gasoline standards to CARBOB.

### Applicability of standards and requirements to CARBOB. All of the standards and requirements in sections 2261, 2262, 2262.3, 2262.4, 2262.5(a), (b), (c) and (e), 2262.6, 2264, 2264.2, 2265, 2267, 2268, 2270, 2271 and 2272 pertaining to California gasoline or transactions involving California gasoline also apply to CARBOB or transactions involving CARBOB. Whenever the term “California gasoline” is used in the sections identified in the preceding sentence, the term means “California gasoline or CARBOB.” Whenever the term “gasoline” is used in section 2265(b)(1), the term means “California gasoline or CARBOB.”

### Determining whether a final blend of CARBOB complies with the standards for California gasoline.

#### General.

##### Applicability. This section (a)(2) governs the determination of whether a final blend of CARBOB complies with the standards for California gasoline that apply when the gasoline is sold or supplied from the production or import facility at which it was produced or imported. Section (a)(6) governs the determination of whether downstream CARBOB that has already been supplied from its production or import facility complies with the applicable cap limits for California gasoline.

##### Where a producer or importer has designated a final blend as CARBOB and has complied with all applicable provisions of this section, the properties of the final blend for purposes of compliance with sections 2262, 2262.3, 2262.4, 2262.5, 2262.6, and 2265  shall be determined in accordance with section (a)(2)(B) or (a)(2)(C) as applicable.

##### If the producer or importer has not complied with all applicable provisions of this section, the properties of the final blend for purposes of the producer's or importer's compliance with the limits for sulfur, benzene, aromatic hydrocarbons, olefins, T50, T90, and oxygen required by sections 2262.3, 2262.5, and 2265 shall be determined without using the CARBOB Model or adding oxygenate to the gasoline, and compliance with the flat limits for Reid vapor pressure and oxygenates required by sections 2262.4, 2262.6, and 2265 shall be determined in accordance with section (a)(2)(B) or (a)(2)(C) as applicable.

#### Determining whether a final blend of CARBOB complies with the standards for California gasoline by use of the CARBOB Model.

##### A producer or importer may elect to have the CARBOB model used in determining whether a final blend designated as CARBOB complies with the standards applicable to California gasoline, by providing the notice in section (b)(1)(C). In this case, the CARBOB limits for the final blend shall be determined in accordance with the “Procedures for Using the California Model for California Reformulated Gasoline Blendstocks for Oxygenate Blending (CARBOB),” as adopted April 25, 2001, last amended August 7, 2008, which is incorporated by reference herein. The CARBOB's compliance with the assigned CARBOB limit for a property shall constitute compliance with the corresponding finished gasoline limit -be it a section 2262 flat limit, Predictive Model flat limit, or (if no designated alternative limit has been established) section 2262 or Predictive Model averaging limit. In addition, where the producer or importer has elected to use the CARBOB model for a given final blend that is not being transferred from its production or import facility during the Reid vapor pressure control period for that facility set forth in section 2262.4(a), the final blend must have a Reid vapor pressure no lower than the value used in the T50 CARBOB model.

##### Notwithstanding section (a)(2)(B)1., where a final blend of CARBOB is sampled and analyzed by a state board inspector in accordance with section 2263 using the methodology in (a)(2)(C), the results may be used to establish a violation of applicable standards for California gasoline.

#### Determining whether a final blend of CARBOB complies with the standards for California gasoline by oxygenate blending and testing. Except as otherwise provided in section (a)(2)(B), the properties of a final blend of CARBOB shall be determined for purposes of compliance with sections 2262, 2262.3, 2262.4, 2262.5, 2262.6, and 2265by adding the specified type and amount of oxygenate to a representative sample of the CARBOB and determining the properties and characteristics of the resulting gasoline in accordance with an applicable test method identified in section 2263(b) or permitted under section 2263(c). Where the producer or importer has in accordance with section (b)(1)(E) designated a range for oxygen from denatured ethanol of 1.8 wt.% to 2.2 wt.% (or a range that is within 1.8 wt.% and 2.2 wt.% and includes 2.0 wt.%), denatured ethanol equal to 5.7 vol. % of the blended volume shall be added; where the designated range for oxygen from denatured ethanol is 2.5 wt.% to 2.9 wt.% (or is within 2.5 wt.% and 2.9% and includes 2.7 wt.%), denatured ethanol equal to 7.7 vol.% of the blended volume shall be added; and where the designated range for oxygen from denatured ethanol is 3.3 wt.% to 3.7 wt.% (or is within 3.3 wt.% and 3.7 wt.% and includes 3.5 wt.%), denatured ethanol equal to 10.0 vol.% of the blended volume shall be added. In all other cases where the designated range for oxygen from denatured ethanol is no greater than 0.4 wt.%, the amount of denatured ethanol added shall be the volume percent that results in an oxygen content at the midpoint of the range of oxygen, based on the following equation:

Vol.% Denatured Ethanol = 620 / [(218.8 / wt.% oxygen) - 0.40]

Where the producer or importer has in accordance with section (b)(1)(E) designated a range of amounts of oxygen that is greater than 0.4 wt.%, or an oxygenate other than denatured ethanol, the oxygenate shall be added in an amount that results in an oxygen content within 0.2 wt.% of the designated minimum oxygen level.

#### Characteristics of denatured ethanol used in determining whether a final blend of CARBOB complies with the standards for California gasoline.

##### Default denatured ethanol characteristics on or after December 31, 2003 when the CARBOB Model is used. Except as provided in section (a)(2)(D)3., where a producer or importer has elected to use the CARBOB Model for a final blend of CARBOB supplied from its production or import facility on or after December 31, 2003, the following default denatured ethanol specifications shall be specified for the CARBOB Model:

| Sulfur content: | 10 parts per million |
| --- | --- |
| Benzene content: | 0.06 volume percent |
| Olefin content: | 0.5 volume percent |
| Aromatic hydrocarbon content: | 1.7 volume percent |

##### Default denatured ethanol characteristics on or after December 31, 2003 when the CARBOB Model is not used. Except as provided in section (a)(2)(D)3., where a producer or importer has not elected to use the CARBOB Model, denatured ethanol used as the oxygenate must have the following properties in determining whether CARBOB complies with the standards applicable to California gasoline when it is supplied from the production facility or import facility on or after December 31, 2003:

| Sulfur content: | 3-10 parts per million |
| --- | --- |
| Benzene content: | 0-0.06 volume percent |
| Olefin content: | 0-0.5 volume percent |
| Aromatic hydrocarbon content: | 0-1.7 volume percent |

##### Producer- or importer-specified characteristics of denatured ethanol used in determining whether a final blend of CARBOB complies with the standards for California gasoline.

###### With respect to a final blend of CARBOB supplied from its production or import facility on or after December 31, 2003, the producer or importer may elect to specify the properties of the oxygenate in accordance with the preceding sentence. Where the producer or importer has elected to use the CARBOB model in connection with the final blend, the maximum value for each property identified in the section (b)(1)(D) notification shall be used for the CARBOB Model. Where the producer or importer has not elected to use the CARBOB model in connection with the final blend, the oxygenate used in oxygenate blending and testing in accordance with section (a)(2)(C)1. must not exceed the maximum value for each property identified in the section (b)(1)(D) notification; that oxygenate's specifications for each property may be under the maximum value for each property identified in the section (b)(1)(D) notification by no more than the following:

| Sulfur content: | 5 parts per million |
| --- | --- |
| Benzene content: | 0.06 volume percent |
| Olefin content: | 0.1 volume percent |
| Aromatic hydrocarbon content: | 1.0 volume percent |

###### Maintaining oxygenate samples for use in compliance testing. A producer or importer who is specifying the properties of the oxygenate used in a final blend of CARBOB in accordance with the preceding section (a)(2)(D)3.a. must maintain at the production or import facility, while the final blend is at the facility, oxygenate meeting the required specifications in quantities that are sufficient to enable state board inspectors to use the oxygenate in compliance determinations.

#### Protocol for determining whether a final blend of CARBOB complies with the standards for California gasoline. The executive officer may enter into a written protocol with any individual producer or importer for the purpose of specifying a alternative method for determining whether a final blend of CARBOB complies with the standards for California gasoline, as long as the executive officer reasonably determines that application of the protocol is not less stringent or enforceable than application of the express terms of section (a)(2)(A)-(D). Any such protocol shall include the producer's or importer's agreement to be bound by the terms of the protocol.

### Calculating the volume of a final blend of CARBOB. Where a producer or importer has designated a final blend as CARBOB and has complied with all applicable provisions of this section, the volume of the final blend shall be calculated for all purposes under section 2264 by adding the minimum designated amount of the oxygenate having the smallest volume designated by the producer or importer. If the producer or importer has not complied with any applicable provisions of this section, the volume of the final blend for purposes of the refiner or producer's compliance with sections 2262, 2262.3, 2262.4, 2262.5, 2262.6 and 2265 shall be calculated without adding the amount of oxygenate to the CARBOB.

### Specifications for a final blend of CARBOB when the CARBOB model is not being used. A producer or importer who has not elected to use the CARBOB model pursuant to section (a)(2)(B) with regard to a final blend of CARBOB may not sell, offer for sale, supply or offer for sale that final blend of CARBOB from its production facility or import facility where the sulfur, benzene, olefin or aromatic hydrocarbon content of the CARBOB, when multiplied by (1 minus the designated maximum volume percent, expressed as a decimal fraction, that the oxygenate will represent after it is added to the CARBOB), results in a sulfur, benzene, olefin or aromatic hydrocarbon content value exceeding the applicable limit for that property.

### Assignment of designated alternative limits for CARBOB and for the oxygenated California gasoline where the producer or importer has elected to use the CARBOB model.

#### Applicability. This section (a)(5) applies where a producer or importer has elected to have the CARBOB model apply in connection with a final blend of CARBOB which is also subject to an averaging compliance option or a Predictive Model averaging compliance option for one or more properties.

#### Assignment of CARBOB designated alternative limit. The producer or importer may assign a CARBOB designated alternative limit for the final blend of CARBOB by satisfying the notification requirements of section (a)(5)(D). In no case shall a CARBOB designated alternative limit be less than the sulfur, benzene, olefin or aromatic hydrocarbon content, or T90 or T50, of the final blend shown by the sample and test of the CARBOB conducted pursuant to section 2270. The CARBOB designated alternative limit shall be treated as the designated alternative limit under section 2262.3(c)(2), and a violation of section 2262.3(c)(2) will exist when the CARBOB exceeds the CARBOB designated alternative limit.

#### Determining the designated alternative limit for the final blend after the CARBOB is oxygenated. Whenever a producer or importer has assigned a designated alternative limit for a final blend of CARBOB, the designated alternative limit for the final blend after the CARBOB is oxygenated shall be determined in accordance with the “Procedures for Using the California Model for California Reformulated Gasoline Blendstocks for Oxygenate Blending (CARBOB),” as adopted April 25, 2001, last amended August 7, 2008, which is incorporated by reference herein. This will be the final blend's designated alternative limit for purposes of compliance with sections 2262.3(c)(3) and 2264(b) and (c).

#### Notification. The producer or importer shall notify the executive officer of the CARBOB designated alternative limit, the designated alternative limit for the final blend after it is oxygenated, and all other information identified in section 2264(a)(2)(A), within the time limits set forth in section 2264(a)(2)(A) and subject to section 2264(a)(3) and (4).

### Determining whether downstream CARBOB complies with the cap limits for California gasoline.

#### Determining whether downstream CARBOB complies with the cap limits for California gasoline through the use of CARBOB cap limits derived from the CARBOB Model. Whenever downstream CARBOB designated for ethanol blending has already been supplied from its production or import facility, the CARBOB's compliance with the cap limits for California gasoline may be determined by applying the CARBOB cap limits in the following table:

| Property | CARBOB Cap Limits |  |
| --- | --- | --- |
| Reid Vapor Pressure1 (pounds per square inch) |  | 5.99 |
| Sulfur Content (parts per million by weight) |  |  |
|  |  | 212 |
| Benzene Content (percent by volume) |  | 1.22 |
| Aromatics Content (percent by volume) |  | 38.7 |
| Olefins Content (percent by volume) |  | 11.1 |
| T50 (degrees Fahrenheit) |  | 2323 |
|  |  | 2373 |
| T90 (degrees Fahrenheit) |  | 335 |

1 The Reid vapor pressure standards apply only during the warmer weather months identified in section 2262.4.

2 CaRFG Phase 3 CARBOB cap limits for sulfur starting December 31, 2011.

3 The first number applies to CARBOB that is subject to the Reid vapor pressure standard pursuant to section 2262.4, and the second number applies to CARBOB that is not subject to the Reid vapor pressure standard.

#### Determining whether downstream CARBOB complies with the cap limits for California gasoline by oxygenate blending and testing. Whenever downstream CARBOB designated for oxygenate blending has already been supplied from its production or import facility, the CARBOB's compliance with the cap limits for California gasoline may be determined by adding the specified type and amount of oxygenate to a representative sample of the CARBOB and determining the properties and characteristics of the resulting gasoline in accordance with an applicable test method identified in section 2263(b) or permitted under section 2263(c). Denatured ethanol used as the oxygenate must have the properties set forth in section (a)(2)(D)2. Where the designated range for oxygen from denatured ethanol is 1.8 wt.% and 2.2 wt.% (or is within 1.8 wt.% and 2.2 wt.% and includes 2.0 wt.%), denatured ethanol equal to 5.7 vol.% of the blended volume shall be added; where the designated range for oxygen from denatured ethanol is 2.5 wt.% and 2.9 wt.% (or is within 2.5 wt.% and 2.9 wt.% and includes 2.7 wt.%), denatured ethanol equal to 7.7 vol.% of the blended volume shall be added; and where the designated range for oxygen from denatured ethanol is 3.3 wt.% to 3.7 wt.% (or is within 3.3 wt.% and 3.7 wt.% and includes 3/5 wt.%), denatured ethanol equal to 10.0 vol.% of the blended volume shall be added. In all other cases where the designated range for oxygen from denatured ethanol is no greater than 0.4 wt.%, the amount of denatured ethanol added shall be the volume percent that results in an oxygen content at the midpoint of the range of oxygen, based on the following equation:

Vol.% Denatured Ethanol = 620 / [(218.8 / wt.% oxygen) - 0.40]

Where the designated a range of amounts of oxygen is greater than 0.4 wt.%, or an oxygenate other than denatured ethanol is designated, the oxygenate shall be added in an amount that results in an oxygen content within 0.2 wt.% of the designated minimum oxygen level.

#### Protocols. A person may enter into a protocol with the executive officer for the purpose of identifying more stringent specifications for the denatured ethanol used pursuant to section (a)(6)(B), or different CARBOB cap limits under section (a)(6)(A), if the executive officer reasonably determines that the specifications or cap limits are reasonably premised on the person's program to assure that the denatured ethanol added to the CARBOB by oxygenate blenders will meet the more stringent specifications.

## Notification to CARB regarding the supply of CARBOB from the facility at which it was produced or imported.

### A producer or importer supplying a final blend of CARBOB from the facility at which the producer or importer produced or imported the CARBOB must notify the executive officer of the information set forth below, along with any information required under section 2265(a)(2) (for a Predictive Model alternative gasoline formulation) or section 2265.1 (for a Predictive Model emissions offsetting formulation, applicable only to producers and importers that produce gasoline). The notification must be received by the executive officer before the start of physical transfer of the final blend of CARBOB from the production or import facility, and in no case less than 12 hours before the producer or importer either completes physical transfer or commingles the final blend.

#### The identity and location of the final blend;

#### The designation of the final blend as CARBOB;

#### If the producer or importer is electing to use the CARBOB model to determine whether the final blend complies with the standards applicable to California gasoline when it is supplied from the production facility or import facility, a statement of that election and

##### Each of the CARBOB limits that will apply to the final blend for properties not subject to the averaging compliance option or the Predictive Model averaging compliance option; and

##### For any property subject to the averaging compliance option or the Predictive Model averaging compliance option, the averaging or Predictive Model averaging limit for the CARBOB (the CARBOB is subject to this limit only if no designated alternative limit is assigned to the CARBOB pursuant to section (a)(5)(B));

#### If the producer or importer is specifying, pursuant to section (a)(2)(D)3., the properties of the oxygenate to be added downstream by the oxygenate blender, a statement of that election, the type of oxygenate, and the oxygenate's specifications for the following properties:

Maximum sulfur content (nearest part per million by weight)

Maximum benzene content (nearest hundredth of a percent by volume)

Maximum olefin content (nearest tenth of a percent by volume)

Maximum aromatic hydrocarbon content (nearest tenth of a percent by volume)

#### The designation of each oxygenate type or types and amount or range of amounts to be added to the CARBOB, and the applicable flat limit, Predictive Model alternative specification or designated emissions offsetting limit. The amount or range of amounts of oxygenate to be added shall be expressed as a volume percent of the gasoline after the oxygenate is added, in the nearest tenth of a percent. For any final blend of CARBOB except one that is subject to Predictive Model alternative specifications or designated emissions offsetting limits, the amount of oxygenate to be added must be such that the resulting California gasoline will have a minimum oxygen content no lower than 1.8 percent by weight and a maximum oxygen content no greater than 2.2 percent by weight. For a final blend of CARBOB that is subject to Predictive Model alternative specifications or designated emissions offsetting limits, the amount or range of amounts of oxygenate to be added must be such that the resulting California gasoline has an oxygen content that meets the oxygen content Predictive Model alternative specification or designated emissions offsetting limits for the final blend.

*    *    *    *

## Documentation required when CARBOB is transferred.

### Required Documentation. On each occasion when any person transfers custody or title of CARBOB, the transferor shall provide the transferee a document that prominently:

#### States that the CARBOB does not comply with the standards for California gasoline without the addition of oxygenate,

#### Identifies the applicable flat limit, Predictive Model alternative specification, or designated emissions offsetting limit, and

#### Identifies, consistent with the notification made pursuant to section (b), the oxygenate type or types and amount or range of amounts that must be added to the CARBOB to make it comply with the standards for California gasoline. Where the producer or importer of the CARBOB has elected to specify the properties of the oxygenate pursuant to section (b)(1)(D), the document must also prominently identify the maximum permitted sulfur, benzene, olefin and aromatic hydrocarbon contents -- not to exceed the maximum levels in the section (b)(1)(D) notification -- of the oxygenate to be added to the CARBOB.

### Compliance by pipeline operator. A pipeline operator may comply with this requirement by the use of standardized product codes on pipeline tickets, where the code(s) specified for the CARBOB is identified in a manual that is distributed to transferees of the CARBOB and that sets forth all of the required information for the CARBOB.

## Restrictions on transferring CARBOB.

### Required agreement by transferee. No person may transfer ownership or custody of CARBOB to any other person unless the transferee has agreed in writing with the transferor that either:

#### The transferee is a registered oxygenate blender and will add oxygenate of the type(s) and amount (or within the range of amounts) designated in accordance with section (b) before the CARBOB is transferred from a final distribution facility, or

#### The transferee will take all reasonably prudent steps necessary to assure that the CARBOB is transferred to a registered oxygen blender who adds the type and amount (or within the range of amounts) of oxygenate designated in accordance with section (b) to the CARBOB before the CARBOB is transferred from a final distribution facility.

### Prohibited sales of CARBOB from a final distribution facility. No person may sell, offer, or supply CARBOB from a final distribution facility where the type and amount or range of amounts of oxygenate designated in accordance with section (b) has not been added to the CARBOB.

## Restrictions on blending CARBOB with other materials.

### Basic prohibition. No person may combine any CARBOB that has been supplied from the facility at which it was produced or imported with any material except:

#### The specified oxygenate.

##### The CARBOB may be blended with oxygenate of the type and amount (or within the range of amounts) specified by the producer or importer at the time the CARBOB was supplied from the production or import facility.

##### Where ethanol is the specified oxygenate and specifications for the ethanol are identified in the product transfer document for the CARBOB pursuant to section (c)(1)(C), only ethanol meeting those specifications may be combined with the CARBOB.

##### Where ethanol is the specified oxygenate and specifications for the ethanol are not identified, only ethanol meeting the standards in section 2262.9(a) may be combined with the CARBOB.

#### Identically-specified CARBOB. The CARBOB may be blended with other CARBOB for which the same oxygenate type, and the same amount (or range of amounts) of oxygen, was specified by the producer or importer at the time the CARBOB was supplied from the production or import facility. However, where specifications for the denatured ethanol to be added to the CARBOB have been established pursuant to section (a)(2)(D)3, it may only be blended with other CARBOB for which the same denatured ethanol specifications have been set.

*    *    *    *

#### Limited amounts of California gasoline containing ethanol. A person may add California gasoline containing ethanol to CARBOB at a terminal or bulk plant if all of the following conditions are met, in which case the resulting mixture will continue to be treated as CARBOB.

##### The gasoline is added to the CARBOB for one of the following operational reasons:

###### The gasoline resulted from oxygenating CARBOB at the terminal or bulk plant during calibration of oxygenate blending equipment; or

###### The gasoline resulted from the unintentional over- or under-oxygenation of CARBOB during the loading of a cargo tank truck at the terminal or bulk plant; or

###### The gasoline was pumped out of a gasoline storage tank at a motor vehicle fueling facility for legitimate operational reasons.

##### The non-oxygenate portion of the gasoline complies with the applicable cap limits for CARBOB in section (a)(6).

##### The resulting mixture of CARBOB has an oxygen content not exceeding 0.1 percent by weight.

###### The oxygen content of the mixture may be determined arithmetically by

[i] 	using the volume of the CARBOB prior to mixing based on calibrated tank readings,

[ii] 	using the volume of the gasoline added based on calibrated meter readings,

[iii] 	using the volume of the denatured ethanol in the gasoline being added based on direct calibrated meter readings of the denatured ethanol if available,

[iv] 	calculating weight percent oxygen of the gasoline being added from volume percent denatured ethanol based on the following formula:

(wt.% oxygen) ≃ 218.8/{[620/(vol.% deEtOH)] + 0.40}, and

[v] 	accounting for any oxygen in the CARBOB tank due to previous additions of gasoline to the tank.

###### If the meter readings described in section (e)(1)(E)3.a.[iii] are not available, the oxygen content of the mixture may be determined arithmetically by

[i] 	using the volume of the CARBOB prior to mixing based on calibrated tank readings,

[ii] 	using the volume of the gasoline added based on calibrated meter readings,

[iii] 	using the oxygen content of the gasoline in weight percent based on sampling and testing of the gasoline for denatured ethanol content in accordance with methods specified in section 2263, and

[iv] 	accounting for any oxygen in the CARBOB tank due to previous additions of gasoline to the tank.

###### In making the determination described in section (e)(1)(E)3.a. or b., the oxygen content of the mixture shall be calculated based on the following formula:

(wt.% oxygen) ≃ [(volume CARBOB)*(wt.% oxygen in CARBOB) + (volume gasoline)*(wt.% oxygen in gasoline] / [(volume CARBOB) + (volume gasoline)].

##### Prior to the mixing, the operator of the terminal or bulk plant notifies the executive officer of the following:

###### The identity and location of the facility at which the mixing will take place;

###### The operational reason for adding the gasoline into the CARBOB;

###### The projected percentage oxygen content of the mixture.

##### The terminal or bulk plant operator maintains for two years records documenting the information identified in section (e)(1)(E)4, and makes them available to the executive officer upon request.

#### Deposit Control Additives that meet the limits specified in sections 2253.4 and 2254, and that are certified pursuant to 2257

#### Additives that a pipeline operator would add for operational purposes, such as drag reducing agent.

#### Non-CARBOB material in a pipeline. Where a person is transporting a tender of CARBOB followed by a tender of non-CARBOB material in a pipeline, or vice versa, the interface between the CARBOB and non-CARBOB material shall be diverted to a separate storage tank and may not be offered, sold, or supplied as CARBOB or as California gasoline.

#### Non-CARBOB material in a storage tank. Where a person is changing from a non-CARBOB material to a CARBOB in a storage tank at a terminal or bulk plant, the resultant mixture shall not exceed the CARBOB cap limits. If this condition is met, the resultant mixture in the tank may be treated as CARBOB. In addition, all persons who are subject to this exception shall maintain, for two years from the date of each occurrence, records demonstrating that the resultant mixture did not exceed the CARBOB cap limits. During the period of required retention, the person subject to this exception shall make any of the records available to the Executive Officer upon request. Whenever such a person fails to provide the foregoing records, the volume of blended material will be presumed to have been blended by the person in violation of section (e)(1). If the resultant mixture does not meet the CARBOB cap limits, no person may offer, sell, or supply it as CARBOB or California gasoline. Where a person is changing from a CARBOB to a non-CARBOB material in a storage tank, no person may offer, sell, or supply the resultant mixture as CARBOB.

#### Non-CARBOB material in a compartment of a cargo tank truck, marine vessel, rail car, or other vessel. Where a person is changing from a non-CARBOB material to a CARBOB in a compartment, the non-CARBOB material shall not exceed 0.25 percent of the compartment's safe fill volume. If this condition is met the resultant mixture in the compartment may be treated as CARBOB. Where a person is changing from a CARBOB to a non-CARBOB material in a compartment, no person may offer, sell, or supply the resultant mixture as CARBOB.

#### Vapor recovery condensate, provided the resulting mixture of CARBOB has an oxygen content not exceeding 0.1 percent by weight.

### Protocols.

#### Protocols covering the changeover in service of a storage tank. Notwithstanding section (f)(1), the executive officer may enter into a written protocol with any person to identify conditions under which the person may lawfully combine CARBOB with California gasoline or other CARBOB during a changeover in service of a storage tank for a legitimate operational business reason. The executive officer may only enter into such a protocol if he or she reasonably determines that commingling of the two products will be minimized as much as is reasonably practical. Any such protocol shall include the person's agreement to be bound by the terms of the protocol.

#### Protocols for blending transmix into CARBOB. Notwithstanding section (e)(1), the executive officer may enter into a written protocol with any person to identify conditions under which the person may lawfully blend transmix into CARBOB which has been supplied from its production or import facility. The executive officer may enter into such a protocol only if he or she reasonably determines that alternatives to the blending are not practical and the blending will not significantly affect the properties of the CARBOB gasoline into which the transmix is added. Any such protocol shall include the person's agreement to be bound by the terms of the protocol.

#### Protocols In Other Situations. Notwithstanding section (e)(1), the executive officer may enter into a written protocol with any person to identify conditions under which the person may lawfully add California gasoline or other CARBOB to CARBOB in a storage tank at a terminal or bulk plant in situations other than those identified in sections (e)(1)(C), (D), or (E), or (e)(2)(A) or (B). The executive officer may enter into such a protocol only if he or she reasonably determines that alternatives to the activity are not practical and the blending will not significantly affect the properties of the CARBOB into which the gasoline or CARBOB is added. The protocol shall include any of the conditions in section (e)(1)(E) that the executive officer determines are necessary and appropriate. Any such protocol shall include the person's agreement to be bound by the terms of the protocol.

#### Protocols for the incidental mixing of non-CARBOB material with CARBOB during the normal and correct operation of a business. Notwithstanding section (e)(1), the Executive Officer may enter into a written protocol with any person to identify conditions under which the person may lawfully mix non-CARBOB materials with CARBOB that has been supplied from a production or import facility. The Executive Officer may enter into such a protocol if he or she reasonably determines that alternatives to the blending are not practical, commingling of the non-CARBOB with CARBOB material will be minimized as much as is reasonably practical, and the blending will not significantly affect the properties of the CARBOB into which the non-CARBOB material is added and will not exceed CARBOB cap limits. If the Executive Officer reasonably determines that the resultant mixture will not be offered, sold, or supplied as CARBOB, he or she may also enter into such a protocol. Any such protocol shall include the person's agreement to be bound by the terms of the protocol.

## Requirements for oxygenate blenders.

### Registration and Certification.

#### Registration. Any oxygen blender must register with the executive officer by March 1, 1996, or at least 20 days before blending oxygenates with CARBOB, whichever occurs later. Thereafter, an oxygenate blender must register with the executive officer annually by January 1. Oxygen blenders must submit the information required in (f)(1)(B) on CARB’s Fuels Blender & Distributor Certifications website: air.arb.ca.gov/FuelsCertifications.

#### Required contents of registration. The registration must include the following:

##### The oxygen blender's contact name, telephone number, principal place of business which shall be a physical address and not a post office box, and any other place of business at which company records are maintained.

##### For each of the oxygen blender's oxygenate blending facilities, the facility name, physical location, contact name, and telephone number.

#### Issuance of certificate. The executive officer shall provide each complying oxygen blender with a certificate of registration compliance no later than June 30. The certification shall be effective from no later than July 1, through June 30 of the following year. The certification shall constitute the oxygen blender's certification pursuant to Health and Safety Code section 43026.

#### Submittal of updated information. Any oxygen blender must submit updated registration information to CARB’s Fuels Blender & Distributor Certifications website: air.arb.ca.gov/FuelsCertifications within 30 days of any occasion when the registration information previously supplied becomes incomplete or inaccurate.

*    *    *    *

## Downstream blending of California gasoline with nonoxygenate blendstocks.

### Basic prohibition. No person may combine California gasoline which has been supplied from a production or import facility with any nonoxygenate blendstock, other than vapor recovery condensate, unless the person can affirmatively demonstrate that (1) the blendstock that is added to the California gasoline meets all of the California gasoline standards without regard to the properties of the gasoline to which the blendstock is added, and (2) the person meets with regard to the blendstock all requirements in this subarticle applicable to producers of California gasoline.

### Exceptions.

#### Protocols. Notwithstanding section (g)(1), the executive officer may enter into a written protocol with any person to identify conditions under which the person may lawfully blend transmix into California gasoline which has been supplied from its production or import facility. The executive officer may only enter into such a protocol if he or she reasonably determines that alternatives to the blending are not practical and the blending will not significantly affect the properties of the California gasoline into which the transmix is added. Any such protocol shall include the person's agreement to be bound by the terms of the protocol.

#### Blending to meet a cap limit. Notwithstanding section (g)(1), a person may add nonoxygenate or oxygenated blendstock to California gasoline that does not comply with one or more of the applicable cap limits contained in section 2262, where the person obtains the prior approval of the executive officer based on a demonstration that adding the blendstock is a reasonable means of bringing the gasoline into compliance with the cap limits.

#### Blending with E85. Notwithstanding section (g)(1), a person may blend E85 with E10 to produce a final blend of E15.

## Restrictions during the RVP season on blending gasoline containing ethanol with California gasoline not containing ethanol.

### Basic prohibition. Within each air basin during the Reid vapor pressure cap limit periods specified in section 2262.4(a)(2), no person may combine California gasoline produced using ethanol with California gasoline produced without using ethanol, unless the person can affirmatively demonstrate that: (A) the resulting blend complies with the cap limit for Reid vapor pressure set forth in section 2262, or (B) the person has taken reasonably prudent precautions to assure that the gasoline is not subject to the Reid vapor pressure cap limit either because of sections 2261(c) or (f) or 2262.4(c)(1) or (c)(3), or because the gasoline is no longer California gasoline.

### Exception. Section (h)(1) does not apply to combining California gasolines that are in a motor vehicle's fuel tank.

Note: Authority cited: Sections 39600, 39601, 43013, 43013.1, 43018 and 43101, Health and Safety Code. Reference: Sections 39000, 39001, 39002, 39003, 39010, 39500, 39515, 39516, 41511, 43000, 43013, 43013.1, 43016, 43018, 43021 and 43101, Health and Safety Code; and Western Oil and Gas Ass’n v. Orange County Air Pollution Control District, 14 Cal.3d 411, 121 Cal. Rptr. 249 (1975).

*    *    *    *

# 2270. Testing and Recordkeeping.

### The requirements of this section (a) shall apply to each producer and importer that has elected to be subject to an averaging limit in section 2262, or to a Predictive Model averaging limit, or to a producer or importer that produces gasoline that has elected to be subject to the Predictive Model emissions offsetting compliance option pursuant to section 2264.2(d). The references to sulfur content shall apply to each producer or importer that has elected to be subject to the section 2262 averaging limit for sulfur, or to a Predictive Model averaging limit for sulfur, or to a producer or importer that produces gasoline that has elected to be subject to the Predictive Model emissions offsetting compliance option pursuant to section 2264.2(d). The references to benzene content shall apply to each producer or importer that has elected to be subject to the section 2262 averaging limit for benzene, or to a Predictive Model averaging limit for benzene, or to a producer or importer that produces gasoline that has elected to be subject to the Predictive Model emissions offsetting compliance option pursuant to section 2264.2(d). The references to olefin content shall apply to each producer or importer that has elected to be subject to the section 2262 averaging limit for olefin content, or to a Predictive Model averaging limit for olefin content, or to a producer or importer that produces gasoline that has elected to be subject to the Predictive Model emissions offsetting compliance option pursuant to section 2264.2(d). The reference to T90 shall apply to each producer or importer that has elected to be subject to the section 2262 averaging limit for T90, or to a Predictive Model averaging limit for T90, or to a producer or importer that produces gasoline that has elected to be subject to the Predictive Model emissions offsetting compliance option pursuant to section 2264.2(d). The references to T50 shall apply to each producer or importer that has elected to be subject to the section 2262 averaging limit for T50, or to a Predictive Model averaging limit for T50, or to a producer or importer that produces gasoline that has elected to be subject to the Predictive Model emissions offsetting compliance option pursuant to section 2264.2(d). The references to aromatic hydrocarbon content shall apply to each producer or importer that has elected to be subject to the section 2262 averaging limit for aromatic hydrocarbon content, or to a Predictive Model averaging limit for aromatic hydrocarbon content, or to a producer or importer that produces gasoline that has elected to be subject to the Predictive Model emissions offsetting compliance option pursuant to section 2264.2(d). The references to oxygen content and RVP shall apply to each producer or importer that produces gasoline that has elected to be subject to the Predictive Model emissions offsetting compliance option pursuant to section 2264.2(d).

### Each producer shall sample and test for the sulfur, aromatic hydrocarbon, olefin, oxygen, and benzene content, RVP (during the RVP regulatory control periods in section 2262.4(b)(2)), T50 and T90 in each final blend of California gasoline which the producer has produced, by collecting and analyzing a representative sample of gasoline taken from the final blend, using the methodologies specified in section 2263. If a producer blends gasoline components directly to pipelines, tankships, railway tankcars or trucks and trailers, the loading(s) shall be sampled and tested for the sulfur, aromatic hydrocarbon, olefin, oxygen, and benzene content, RVP (during the RVP regulatory control periods in section 2262.4(b)(2)), T50 and T90 by the producer or authorized contractor. The producer shall maintain, for two years from the date of each sampling, records showing the sample date, identity of blend sampled, container or other vessel sampled, final blend volume, sulfur, aromatic hydrocarbon olefin, oxygen, and benzene content, RVP, T50 and T90. All gasoline produced by the producer and not tested as California gasoline by the producer as required by this section shall be deemed to have a sulfur, aromatic hydrocarbon, olefin, oxygen, and benzene content, RVP (during the RVP regulatory control periods in section 2262.4(b)(2)), T50 and T90 exceeding the applicable averaging limit standards specified in section 2262, or exceeding the comparable Predictive Model averaging limits if applicable, or exceeding the designated emissions offsetting limits, unless the importer demonstrates that the gasoline meets those standards and limits.

### Each importer shall sample and test for the sulfur, aromatic hydrocarbon, olefin and benzene content, T50 and T90 in each final blend of California gasoline which the importer has imported by tankship, pipeline, railway tankcars, trucks and trailers, or other means, by collecting and analyzing a representative sample of the gasoline, using the methodologies specified in section 2263. The importer shall maintain, for two years from the date of each sampling, records showing the sample date, product sampled, container or other vessel sampled, the volume of the final blend, sulfur content, aromatic hydrocarbon, olefin and benzene content, T50 and T90. All gasoline imported by the importer and not tested as California gasoline by the importer as required by this section shall be deemed to have a sulfur, aromatic hydrocarbon, olefin and benzene content, T50 and T90 exceeding the applicable averaging limits standards specified in section 2262, or exceeding the comparable Predictive Model averaging limit(s) if applicable, or exceeding the designated emissions offsetting limits, unless the importer demonstrates that the gasoline meets those standards and limit(s).

### A producer or importer shall provide to the executive officer any records required to be maintained by the producer or importer pursuant to this section within 20 days of a written request from the executive officer if the request is received before expiration of the period during which the records are required to be maintained. Whenever a producer or importer fails to provide records regarding a final blend of California gasoline in accordance with the requirements of this section, the final blend of gasoline shall be presumed to have been sold by the producer or importer in violation of the applicable averaging limit standards in section 2262, or the Predictive Model averaging limit(s), or exceeding the designated emissions offsetting limits, to which the producer or importer has elected to be subject.

### The executive officer may enter into a protocol with any producer or importer for the purpose of specifying alternative sampling, testing, recordkeeping, or reporting requirements which shall satisfy the provisions of sections (a)(2) or (a)(3). The executive officer may only enter into such a protocol if s/he reasonably determines that application of the regulatory requirements under the protocol will be consistent with the state board's ability effectively to enforce the averaging limit standards in section 2262, the averaging limit compliance requirements in section 2262.3(c), the Predictive Model averaging limit(s), and the Predictive Model emissions offsetting compliance option requirements in section 2265.1. Any such protocol shall include the producer's or importer's agreement to be bound by the terms of the protocol.

### For each final blend which is sold or supplied by a producer or importer from the party's production facility or import facility, and which contains volumes of gasoline that party has produced and imported and volumes that the party neither produced nor imported, the producer or importer shall establish, maintain and retain adequately organized records containing the following information:

#### The volume of gasoline in the final blend that was not produced or imported by the producer or importer, the identity of the person(s) from whom such gasoline was acquired, the date(s) on which it was acquired, and the invoice representing the acquisition(s).

#### The sulfur, benzene, aromatic hydrocarbon, olefin and benzene content, T50 and T90 of the volume of gasoline in the final blend that was not produced or imported by the producer or importer, determined either by (A) sampling and testing, by the producer or importer, of the acquired gasoline represented in the final blend, or (B) written results of sampling and test of the gasoline supplied by the person(s) from whom the gasoline was acquired.

### A producer or importer subject to this section (b) shall establish such records by the time the final blend triggering the requirements is sold or supplied from the production or import facility, and shall retain such records for two years from such date. During the period of required retention, the producer or importer shall make any of the records available to the executive officer upon request.

## In the event a producer or importer sells, offers for sale, or supplies, in California, gasoline which the producer claims is not California gasoline, such gasoline shall be presumed to exceed the standards that would be applicable pursuant to this subarticle if it was California gasoline. The producer or importer shall maintain, for two years from the date of any sale or supply of such gasoline, records demonstrating that the gasoline was not California gasoline, or that it complied with all of the standards of this subarticle 2, when it was sold or supplied by the producer.

Note: Authority cited: Sections 39600, 39601, 43013, 43013.1, 43018 and 43101, Health and Safety Code. Reference: Sections 39000, 39001, 39002, 39003, 39010, 39500, 39515, 39516, 41511, 43000, 43013, 43013.1, 43016, 43018 and 43101, Health and Safety Code; and Western Oil and Gas Ass'n. v. Orange County Air Pollution Control District, 14 Cal.3d 411, 121 Cal. Rptr. 249 (1975).

# 2271. Variances.

## Applications for variances. Any person who cannot comply with the standards or compliance requirements set forth in sections 2262, 2262.3, 2262.4, 2262.5, 2262.6, or 2265.1 because of reasons beyond the person's reasonable control may apply to the executive officer for a variance. Except for emergency variances as provided in section (h), the application shall be accompanied by a fee of $6700.00 to cover the costs of processing the variance. If the applicant withdraws the application before the variance hearing is held, $4100.00 of the fee shall be refunded. The application shall set forth:

### The applicable section(s) from which the variance is sought;

### The specific grounds upon which the variance is sought;

### The proposed date(s) by which compliance with the provisions of the applicable section(s) will be achieved; and

### A compliance plan reasonably detailing the method by which compliance will be achieved. That proposed compliance plan shall include increments of progress (i.e., specific events and dates) that describe periodic, measurable steps toward compliance during the proposed term of the variance.

### Notices and public hearings for variances. Upon receipt of an application for a variance containing the information required in section (a), the executive officer shall hold a hearing to determine whether, or under what conditions and to what extent, a variance from the requirements of the applicable section(s) is necessary and will be permitted. Notice of the time and place of the hearing shall be sent to the applicant by certified mail not less than 20 days prior to the hearing. Notice of the hearing shall also be submitted for publication in the California Regulatory Notice Register and sent to every person who requests such notice, not less than 20 days prior to the hearing.

### Treatment of confidential information. Information submitted to the executive officer by a variance applicant may be claimed as confidential. Information claimed as confidential shall be handled in accordance with the procedures specified in Title 17, California Code of Regulations (CCR), sections 91000 to 91022 except that: (A) at the time the information is submitted, the submitter must provide accompanying documentation in support of the claim of confidentiality, including the documentation identified in section 91022(c), and (B) for the purposes of this section, the time period specified in section 91022(e)(2) is 10 days instead of 21 days. The executive officer may consider such confidential information in reaching a decision to grant or deny a variance.

## Public participation in the variance process. At least 20 days prior to the hearing, the application for the variance shall be made available to the public for inspection. Interested members of the public shall be allowed a reasonable opportunity to submit written and oral testimony at the hearing and their testimony shall be considered.

## Necessary findings for granting variances. The decision to grant or deny a variance shall be based solely upon substantial evidence in the record of the variance proceeding. No variance shall be granted unless the executive officer makes all of the following findings:

### That, because of reasons beyond the reasonable control of the applicant, requiring compliance with the applicable section(s) would result in an extraordinary economic hardship;

### That the public interest in mitigating the extraordinary hardship by issuing the variance outweighs the public interest in avoiding any increased emissions of air contaminants which would result from issuing the variance; and

### That the compliance plan proposed by the applicant can reasonably be implemented and will achieve compliance as expeditiously as possible.

## Factors to be considered in making the necessary findings for granting variances. In making the findings specified in section (d), the factors set forth below shall be considered. It is the responsibility of the applicant to provide the information necessary to adequately evaluate these factors.

### Regarding the finding specified in section (d)(1):

#### To demonstrate that noncompliance is “beyond the reasonable control of the applicant,” the applicant must demonstrate that reasonably diligent and timely efforts to achieve compliance have been made. Where a variance is sought due to a breakdown, the applicant shall demonstrate that the breakdown could not have been prevented or mitigated by the application of standard industrial practices. “Standard industrial practices” means elements of design, methods of operation, and levels of oversight and maintenance that are regarded as generally accepted practice in the applicant's type of business.

#### To demonstrate that requiring compliance would result in an “extraordinary economic hardship,” the applicant must make a substantial showing that no alternative to a variance would eliminate or mitigate the need for a variance. Potential alternatives that the applicant shall address include the following: 1. obtaining complying gasoline from outside sources, or obtaining blending materials that would allow production of complying gasoline, 2. using the applicable California Predictive Model (as specified in Title 13, CCR, section 2265) to maximize the production of complying gasoline, or to minimize the degree of noncompliance, through the use of a Predictive Model alternative gasoline formulation, and 3. electing to use the Predictive Model emissions offsetting compliance option. The applicant shall compare the economics of operations without a variance, for the period over which the variance is proposed, with the economics of operations after the variance compliance plan has been implemented (e.g., the economic hardship during the term of the variance shall be measured against the eventual cost of long-term compliance.) The operations may include facets of the applicant's business other than gasoline operations, if those facets are directly affected by the ability to conduct the gasoline business. An applicant may also address any supply shortages that could result from the failure to grant a variance and the economic effects of such shortages on the persons who do, or could, receive gasoline from the applicant.

*    *    *    *

Note: Authority cited: Sections 39600, 39601, 43013, 43013.1, 43013.2, 43018 and 43101, Health and Safety Code. Reference: Sections 39000, 39001, 39002, 39003, 39010, 39500, 39515, 39516, 40000, 41511, 43000, 43013, 43013.1, 43013.2, 43016, 43018 and 43101, Health and Safety Code; and Western Oil and Gas Ass'n. v. Orange County Air Pollution Control District, 14 Cal.3d 411, 121 Cal. Rptr. 249 (1975).

*    *    *    *

# 2272. CaRFG Phase 3 Standards for Small Refiners.

## CaRFG Phase 3 standards for qualifying small refiners. In place of the CaRFG Phase 3 standards set forth in section 2262, a small refiner may elect to have a final blend of California gasoline supplied from the small refiner's refinery subject to the “small refiner CaRFG Phase 3 standards,” which are identical to the CaRFG Phase 3 standards in section 2262 except that:

(i) 	the flat limit for benzene content is 1.00 percent by volume (vol.%) instead of 0.80 vol.%,

(ii) 	the flat limit for aromatics content is 35.0 vol.% instead of 25.0 vol.%,

(iii) 	the flat limit for T50 is 220° F. instead of 213° F, and

(iv) 	the flat limit for T90 is 312° F. instead of 305° F. This election may only be made if the small refiner has been issued a currently effective certification pursuant to section (b) and the gasoline qualifies for treatment under section (c).

## Certification of small refiners.

### A small refiner wishing to produce gasoline subject to this section shall submit to the executive officer an application for certification on the Air Resources Board's ARB/SSD/CPB Form 00-3-1, for each of the small refiner's California refineries. The application shall be executed by a responsible corporate officer under penalty of perjury.

### The small refiner's application shall set forth:

[A] 	the crude oil capacity of the refinery since January 1, 1978;

[B] 	the crude oil capacities of all the refineries in California and the United States which are owned or controlled by, or under common ownership or control with, the small refiner since September 1, 1988; and

[C] 	data demonstrating that the refinery has the capacity to produce liquid fuels by distilling petroleum

.

### Within 30 days of receipt of the application, the executive officer shall grant or deny it in writing. The executive officer shall grant the application if he or she determines that:

[A] 	the application contains all of the information identified in sections (b)(1) and (2) above, and

[B] 	the applicant meets the definition of small refiner. Any denial of an application shall include a statement of the reasons for denial.

## Criteria for qualifying gasoline. Gasoline shall only be subject to treatment under this section if the small refiner demonstrates all of the following:

### The gasoline was produced by the small refiner at the small refiner's California refinery.

### The gasoline was supplied from the small refiner's California refinery in a calendar quarter in which 25 percent or more of the gasoline that was produced by the small refiner and that was supplied from the refinery in the calendar quarter was refined at the small refinery from crude oil. The volume of oxygenates in the gasoline shall not be counted in making this calculation. The period from December 31, 2003 through March 31, 2004 shall be treated as a calendar quarter under this section (c)(2).

### For the period December 31, 2003, through December 31, 2004, and for each subsequent calendar year, the gasoline was supplied from the small refiner's California refinery before the full qualifying volume of gasoline produced by the small refiner had been supplied from the refinery during that period or year. In calculating the volume of gasoline supplied from the refinery, the volume of oxygenates in the gasoline shall not be counted. Gasoline that is designated by the small refiner as subject to all of the CaRFG Phase 3 standards in section 2262,and is reported to the executive officer pursuant to a protocol entered into by the small refiner and the executive officer, shall not be counted against the qualifying volume.

### At the time the gasoline was supplied from the small refiner's refinery, the small refiner met the definition of a small refiner.

### The excess emissions of hydrocarbons, oxides of nitrogen, and potency-weighted toxics are offset pursuant to section 2282, title 13, California Code of Regulations. The excess emissions from gasoline subject to the small refiner CaRFG Phase 3 standards are: 0.0206 pounds of exhaust hydrocarbons per barrel, 0.0322 pounds of oxides of nitrogen per barrel, and the potency-weighted toxic emissions equivalent of 0.0105 pounds of benzene per barrel.

## Compliance with applicable federal RFG requirements. Any small refiner subject to this section shall comply with all applicable requirements of the federal reformulated gasoline regulations in 40 CFR Part 80 Subpart D, commencing with § 80.40.

## Additional reporting requirements for small refiners.

### In addition to the requirements of section 2270, each small refiner who qualifies for treatment under this section shall submit to the executive officer reports containing the information set forth below for each of the small refiner's California refineries, starting on the date on which a small refiner supplies from its refinery gasoline subject to the small refiner CaRFG Phase 3 standards. The reports shall be executed in California under penalty of perjury, and must be received within the time indicated below. December 31, 2003 through January 31, 2004 shall be treated as a month.

#### The quantity of all gasoline, produced by the small refiner, that is supplied from the small refinery in each month, within 15 days after the end of the month, the quantity of all such gasoline that is California gasoline subject to the small refiner CaRFG3 standards, and the quantity of all such gasoline that is California gasoline not subject to the small refiner CaRFG3 standards;

#### The identity and volume of each oxygenate contained in the gasoline described in section (e)(1)(A) above, within 15 days after the end of the month;

#### For each calendar quarter, a statement whether 25 percent or more of the gasoline that was produced by the small refinery and that was supplied from the refinery in the calendar quarter was refined at the small refinery from crude oil, within 15 days after the close of such quarter;

#### The date, if any, on which the small refiner completes transfer from its small refinery in the period December 31, 2003 through December 31, 2004, and in each subsequent calendar year, of the small refiner's qualifying volume of gasoline produced by the small refiner, calculated as described in section (c)(3), within 5 days after such date;

#### Within 10 days after project completion, any refinery addition or modification which would affect the qualification of the refiner as a small refiner pursuant to the definition in section 2260; and

#### Any change of ownership of the small refiner or the small refiner's refinery, within 10 days after such change of ownership.

Note: Authority cited: Sections 39600, 39601, 43013, 43018 and 43101, Health and Safety Code. Reference: Sections 39000, 39001, 39002, 39003, 39010, 39500, 39515, 39516, 40000, 41511, 43016, 43018 and 43101, Health and Safety Code; and Western Oil and Gas Ass'n. v. Orange County Air Pollution Control District, 14 Cal.3d 411, 121 Cal. Rptr. 249 (1975).

*    *    *    *

2282. Aromatic Hydrocarbon Content of Diesel Fuel.

*    *    *    *

## Small Refiner Diesel Fuel.

*    *    *    *

### Offsetting Excess Emissions From Gasoline Subject to the Small Refiner CaRFG Phase 3 Standards.

#### Annual elections. No later than December 22 of each calendar year starting with 2002, a small refiner may by notification to the executive officer make the following elections:

*    *    *    *

Note: Authority cited: Sections 39600, 39601, 43013, 43018 and 43101, Health and Safety Code; and Western Oil and Gas Ass'n. v. Orange County Air Pollution Control District, 14 Cal. 3d 411, 121 Cal. Rptr. 249 (1975). Reference: Sections 39000, 39001, 39002, 39003, 39010, 39500, 39515, 39516, 41511, 43000, 43013, 43016, 43018 and 43101, Health and Safety Code; and Western Oil and Gas Ass'n. v. Orange County Air Pollution Control District, 14 Cal. 3d 411, 121 Cal. Rptr. 249 (1975).